SWANFIX.SYS, A SOLUTION TO USING THE SWAN 486DB WITH QEMM-386

The Swan Computers 486DB, a machine which can run at 66 Mhz, has a bug

in its BIOS.  When you use Function 87 of Interrupt 15, the BIOS call which

is used to move data into extended memory, it does not wait long enough for

the 8042 to enable the A20 address line (the address line which must be

enabled in order to use memory above one megabyte).

The 8042 is the keyboard controller chip, a port of which is used to

enable/disable address line A20. When you run this Swan computer at the 66

MHz speed, it does not give the 8042 enough time to honor the BIOS's request

to enable line A20.  Therefore, data is not moved reliably into extended

memory, and later attempts to read data from these memory areas (all odd

megabytes) yield incorrect data, causing the system to fail.

This device driver, Swanfix.sys, hooks Int 15, and, when a function 87

comes in, does an enable of A20 so that by the time the BIOS enables A20, it

will be the second time.  Therefore, by the time any attempt to access memory

(which requires enabled A20) occurs, the line is ready.

Load this driver in the CONFIG.SYS before QEMM386.SYS:

device=c:\qemm\swanfix.sys

device=c:\qemm\qemm386.sys ram st:m

The reason you don't encounter this problem with EMM386.EXE is that EMM386

relies upon HIMEM.SYS, not the BIOS, to move to extended memory, so it doesn't

encounter this bug in the BIOS.

At this time, we know of only one machine which has this problem, but it

is possible that others exist. If your machine crashes upon loading QEMM-386

when in clock-doubled mode, try Swanfix.  Since QEMM-386 takes over this BIOS

function once it has been turned on, using Swanfix is completely free of any

performance degradation.  Quarterdeck is interested in hearing about any other

machines for which this driver is the solution.

************************************************************************

*This technical note may be copied and distributed freely as long as it*

*is distributed in its entirety and it is not distributed for profit.  *

*          Copyright (C) 1993 by Quarterdeck Office Systems            *

************************ E N D   O F   F I L E *************************