���������������������Ŀ

�  From: JON ANDERSON �

�����������������������Ĵ                     ����������������������������

�    Date: 04-10-91   �        Used by Permission.

�����������������������

Now, the fax tsr thing:  Here's a well-documented bat file which uses

bitfax v3.0's TSR form , WAIT.com, and Chuck Eglinton's REMCTL58.zip

to do neat thing:

Batch file follows:

@echo off

goto start

REM ***************** Documentation *******************************

REM

REM ***Purpose: Allow use of FAX card, modem, &/or answering maching

REM    on a single line, without any switching-device

REM

REM ***Requires: 1) Charles Eglinton's excellent shareware utility

REM              called REMOTE CONTROL (latest is REMCTL58.ZIP)

REM           2) Todd Miller's great DOS utility: WAIT.com

REM              available as WAITBAT.ZIP

REM           3) DesqView

REM           4) FAX card with TSR-type software, eg: BitFAX v3.0

REM     Also useful but not required:

REM           5) David G. Thomas' TAME*.ZIP, a DesqView utility to

REM              enhance performance

REM

REM ***Why:

REM

REM Why want 2 separate 1-ring calls to activate FAX? Because having

REM a 1-call requirement obligates the FAX to wait the time defined

REM in the WAIT.com switch line...anyone calling voice will get FAX!

REM So, having a requirement of 2 means one can reset the sequence

REM merely by calling again and ringing a few rings!

REM Also, its unlikely that an unknowing outside party would hangup

REM twice with 1-ring each time.

REM

REM ** How to setup:

REM          Just read thru this bat file and make any directory/drive

REM          changes to be consistent with your own system.  My own

REM          approach, for clarity, is to include all drive/directory

REM          changes to be written here, even ifnec for the logic

REM          of the batch file... this makes things clear, and, as a

REM          habit, lets one move steps around within a bat file with

REM          less chance for the file 'losing its way' on execution.

REM

REM      Setup DV's window : ans. 'Y' to "Close on Exit? ".

REM      DV's window must be setup for communications, i.e, it can NOT

REM      be allowed to swap out!

REM

REM * By Jon Anderson, Boston Gas BBS

REM   617-235-6303  * USR DS * 24hrs/day * Anes/Med/Science/Engineering

REM

REM  **************************************************************

REM

REM   You're free, of course, to edit-out all REM'd lines, which are

REM   ignored by DOS.

REM

REM ****************** end of documentation ***********************

:start

c:\dv\dvansi

e:

c:\tame\tame-res

:TOP

REM remove any fax tsr from any earlier event:

cd\bitfax

RMV_RECV

REM this makes an errorlevel dep. on how many rings before hangup:

:cycle

cd\remctl58

RC 3 ATS0=0

IF ERRORLEVEL 3 GOTO 3

IF ERRORLEVEL 2 GOTO 2

IF ERRORLEVEL 1 GOTO HALFWAY

IF ERRORLEVEL 0 GOTO ABORT

GOTO TOP

:HALFWAY

REM The wait.com used here will show the errorlevel made by RC.com:

wait 00:00:05

cd\remctl58

REM Gets here ONLY if it was the 2nd 'single ring'.. Anything other than

REM a single prev. ring goes to TOP or ABORT (& same via :2 & :3)

RC 3 ATS0=0

IF ERRORLEVEL==3 GOTO 3

IF ERRORLEVEL 2 GOTO 2

IF ERRORLEVEL 1 GOTO 1

IF ERRORLEVEL 0 GOTO ABORT

GOTO TOP

:1

REM Now here 'cause it was the 2nd single ring...

cd\bitfax

recvfax 3

REM must keep 1 min. here, as recvfax is resident,will keep control

REM as long as fax is comming in ??

c:\wait 00:05:00

if errorlevel 1 goto end

GOTO TOP

:2

REM Put anything you want done on 2 ring hangups here

GOTO TOP

:3

REM Put anything you want done on 3 ring hangups here

GOTO TOP

:ABORT

:end

REM Let's see what TAME did, by the way...:

c:\tame\tame /status

REM and change prompt in case you CTRL-C out of the bat file to

REM keep the window open:

REM (and change prompt to be more useful...)

echo                           *

echo Hit CTRL-C to keep window open, but NO FAX's CAN BE REC'D!...

echo                           *

wait 00:00:10

if errorlevel 1 goto stay

exit

:stay

set prompt=-$p-$g

-----

I'm sure Jon spent a fair amount of time creating the wondeful Batch File

and I'd like to pass on my gratitude to him, for allowing the rest of us

to take advantage of his creative imagination!

-----