This is a Freely Distributable File.

In simple terms, you can do anything you like with it, including make money

off it as long as you promise to mention my name with your use of my info.

SupraFan.doc  Copyright(C) 1993, January 5th  ProgrAmiga

Hello!  My name is Jason, and this is my first hardware diagram release.

I have released other software items that you might or might not have seen.

Anyhow, on with the project!

What I noticed after buying my new SupraFaxModem V.32bis was how small it

was!  It's much smaller than my USR 14.4K Dual and had a nifty multi-LED

display.  On the other hand this vast jump in micro-modems had a

side-effect.  The thing runs REALLY HOT!  It works fine for 5-6 hours but

then it would start skipping every-other letter (over-heat).  So I took a

peek at the modem to see if there was any easy way to try to cool it...

Ideas included adding more heat-sinks to taking it out of it's casing.

Well, after researching different cooling methods I figured some kind of

ventilation was needed.  I took out my nifty digital thermometer and

checked the temp inside.  It went past the 110 degree limit on it.  So I

drilled a handfull of holes on the top in a cluster towards the center with

4 holes in each corner on the bottom to hopefully start a convection

current of air.  Stuck the thermometer back in and it held at 105 degrees.

Fine for most people, but according to my education in electronics

(however poor that is) I said it had to be a bit cooler until down to at

least 90 degress or below.  So next idea was a fan.  The fan posed a couple

of problems though.   Most mini-fans are 12VDC and the modem uses a 9VAC

power supply.  Also I wanted it to turn and off along with the modem as

well as not interfere with the modem's function.  This meant it HAD to be

connected after the switch (to turn on and off) and before the +5VDC bridge

rectifier (to not interfere with the chip's functions).  I had to then

change 9VAC to 12VDC which took only a few moments to figure out.. here's

the result:

Items needed:  (W/ Radio Shack Catalog #'s)

(1) EXPERIMENTER'S BOX WITH PC BOARD                270-0283

(1) 1 9/16" DC BRUSHLESS MICRO FAN                  273-0244

(1) SILICON BRIDGE RECTIFIER 200V, 1A               276-1161

(2) 100 MFD ELECTROLYTIC CAPACITOR                  272-1028

After building this simple circuit, you can hook the AC lines to the modem.

Hook one AC line to the rear where the electricity comes in.. hook it to the

rearmost solder joint.  Hook the other one to the switch solder joint in

the front... hook it to the one that's the closest to the front.

DON'T CONFUSE THE TWO GROUNDING ONES AS PART OF THE SWITCH!  They are just

holding the switch in place.. the ones to look at are 3 in a row... use the

first from the from the front one.

Do not replace the fan with anything else.. the modem only has a 650 mA

power supply, and this is the smallest fan I've seen yet. (100 mA) not

leaving much left for the modem's inner workings!  Also, any change in

capacitors can cause a drop in voltage causing the fan not to run (needs

at least 10 Volts).  You can use any rectifier, but this seemed the smallest

and probably the most accurate ac-dc converter for this power-range.

If you like this small project, feel free to write to my permanant

address at:

Jason Ellingson

2114 McMenemy Road North

Maplewood, MN  55117-2016

Expect no responses really fast, I'll try as quick as I can but I'm in the

Air Force, so it'll be up to Uncle Sam as to my free-time to write.