INSTALLING HS/LINK EXTERNAL PROTOCOL

IN SPITFIRE BBS VERSION 3.0

Please refer to the SPITFIRE manual for detailed information on

installation of external protocols.  This information was tested

with HS/Link betas posted on 11/19/91 and SPITFIRE version 3.0.

The following installation steps should work for most folks:

SPITFIRE allows several different external protocols, which must be

lettered sequentially starting with A.  If HS/Link is the first

external protocol you install, it will be letter A.  If it is the

second external protocol, it will be B, and so forth.  In this

document we will assume that HS/Link is the first external protocol

to be installed.  If you already have other external protocols

installed, you will need to change the batch file names

accordingly.  Refer to the spitfire manual chapter 18 for more

details on this subject.

Load up spitfire and press ALT-P.

Check that "<A> External Transfer Protocols" is set to YES.

Check that "<D> Display Files Path" is set to C:\SF\DISPLAY.

Check that "<E> External Transfer Path" is set to C:\SF\EXTNOD1. If

you change this setting you will have to make changes in the

provided batch files.

Place a copy of HSLINK.EXE into C:\SF\EXTNOD1.  Also copy the

supplied SFEXTDNA.BAT and SFEXTUPA.BAT files into the C:\SF\EXTNOD1

directory.  Note that these files must be renamed if you already

have other external protocols installed.

Use a text editor to edit or create the file

C:\SF\DISPLAY\SFEXTDN.BBS. Insert the following line into this

file:

<A> HS/Link,BATCH

Use a text editor to edit or create the file

C:\SF\DISPLAY\SFEXTUP.BBS. Insert the following line into this

file:

<A> HS/Link,BATCH

This should get you set up in under 10 minutes.

Note for systems running locked COM ports:

We have found that if you use a locked DTE of 9600 or above, you

will need to have a NS16550AN UART installed.  Also, for rock solid

performance you should add the -HS (Handshake Slow) switch to your

'SFEXTDNA.BAT' and 'SFEXTUPA.BAT' batch files.

You can edit SFEXTDNA.BAT and SFEXTUPA.BAT to include options on

the HSLINK command line, but most folks won't need to change

anything. If your system can't keep up with the COM port while

accessing the disk drive, you may want to add -HS to the command

line in each file. If you want to experiment, there are a number of

optional parameters you can read about in HSLINK.DOC and

HSLINK.NEW.

Contents of SFEXTDNA.BAT

------------------------

REM HS/LINK BATCH DOWNLOAD FOR SPITFIRE 3.0  (SFEXTDNx.BAT)

ECHO OFF

ECHO EXECUTING EXTERNAL PROTOCOL # 1

CD\SF\EXTNOD1

HSLINK -P%2 -NU %3 %4 %5 %6 %7 %8 %9

IF EXIST %3 ERASE %3

IF EXIST %4 ERASE %4

IF EXIST %5 ERASE %5

IF EXIST %6 ERASE %6

IF EXIST %7 ERASE %7

IF EXIST %8 ERASE %8

IF EXIST %9 ERASE %9

CD\SF

Contents of SFEXTUPA.BAT

------------------------

REM HS/LINK BATCH UPLOAD FOR SPITFIRE 3.0  (SFEXTUPx.BAT)

ECHO OFF

ECHO EXECUTING EXTERNAL PROTOCOL # 1

CD\SF\EXTNOD1

HSLINK -P%2

IF EXIST %3 COPY %3 %4

IF EXIST %5 COPY %5 %4

IF EXIST %6 COPY %6 %4

IF EXIST %7 COPY %7 %4

IF EXIST %8 COPY %8 %4

IF EXIST %9 COPY %9 %4

IF EXIST %3 ERASE %3

IF EXIST %5 ERASE %5

IF EXIST %6 ERASE %6

IF EXIST %7 ERASE %7

IF EXIST %8 ERASE %8

IF EXIST %9 ERASE %9

CD\SF

I hope these instructions help, but remember, I can't take

responsibility for any damages resulting from these instructions.

If you get stuck, it always helps to read the DOC files that come

with SPITFIRE and with HS/Link.  If all else fails, call up the

Tool Shop and ask for help on the Support Conference.  Either I

will help you, or somebody else will know the answer and give you a

hand.

Good luck!

Samuel H. Smith

Sysop of The Tool Shop

Phone number         Modem type                Access

--------------    ---------------------   -------------------

(818) 891-3772    US Robotics HST 9600    Contributors only

(818) 891-1344    Hayes-V series 9600     Contributors only

(818) 891-6780    US Robotics 2400        Free to all