---------------------------------------------------------------------------

deltaComm Development, PO Box 1185, Cary, NC  27512  USA

(919)-460-4556 voice, (919)-460-4531 FAX, (919)-481-9399 BBS

Telix Copyright (C) 1986,87,88,89,90,91,92,93 by deltaComm Development

---------------------------------------------------------------------------

October 13th, 1993 -- Notes on Telix 3.21 patch:

This patch fixes a minor problem with regard to Telix sometimes not

being able to turn off the 16550 FIFO, which can cause problems with

other ill-behaved software such as Prodigy's software and certain fax

software.  This patch does NOT correct the problem in which a warm

boot on certain AMI BIOS's can cause the comm port not to be seen or

recognized at all.  Only a BIOS replacement can correct this problem.

Thanks to our German distributors (Elsa GmbH, Aachen), specifically

Elsa's software engineers led by Svan Geuer, who discovered an

undocumented (and highly illogical) dependency with regard to the

order in which certain serial port registers are restored when

closing the comm port.  This patch corrects the problem, enabling

Telix to correctly restore the state of the 16550 FIFO under all

known circumstances.

To apply this patch:

1) This patch must be applied to the UNBRANDED, REGISTERED copy of

Telix.  It cannot be applied to branded copies, or to the

shareware version of Telix.  The unbranded TELIX.EXE file can be

found on the distribution diskette in the file TLX321-1.ZIP and

is exactly 274,078 bytes, dated 02-05-93.

2) Place the files PATCH.EXE and PATCH.RTP in the same directory as

the unbranded TELIX.EXE, and run the program PATCH.EXE from that

subdirectory.

3) The resultant copy of TELIX.EXE may then be branded and used.

For those inclined to ask, Telix restored the comm registers in the

inverse order of which they were initialized.  This included

restoring the Interrupt ID (IID) register prior to the Line Control

Register (LCR).  It was found that (although it shouldn't matter), on

some systems the restoration of the LCR register after the IID

register would sometimes reactivate the FIFO (or possibly the

contents of the initialized LCR would prevent the FIFO from being

deactivated at all -- we're really not sure).  By restoring the LCR

to the original state before restoring the IID register, tests have

confirmed that the 16550 FIFO is always thus restored.  Don't ask

why, since we don't know -- this behaviour isn't documented in the

National Semiconductor specs *or* their sample code (their sample

code also restores the IID before the LCR).

Jeff Woods

October 13th, 1993