ComRing - packet driver for "network" based on serial ports

version 1.13

Copyright 1997 by Jan Bobrowski

This is free software. Use it at your own risk.

MOTIVATION

The intention of ComRing is to connect more than two computers

through	ordinary serial ports. ComRing cable allows you to

build "network" based on serial ports using one port per

computer. It is slow but cheap.

HARDWARE

Serial port was developed for computer-modem connection. It is

widely used to connect two computers using null-modem cable.

It consists of two units: trasmitter and receiver. They may be

connected to different computers (?). The basic idea of ComRing

is to organize computers in logical form of ring. Data will flow

in fixed direction.

A �����> B

^	 �

�	 �

�	 v

D <����� C

You should prepare 3-wire ComRing cable:

DB9 DB25

DB25					2   3	RxD <��Ŀ

_____________________________		3   2	TxD ��Ŀ�

1 \ . T R . . . G . . . . . . / 13	5   7	GND ��ͳ�ͻ

14 \ . . . . . . . . . . . . / 25		       �� �

-----------------------		2   3	RxD <��ٳ �

3   2	TxD ��Ŀ� �

DB9					5   7	GND ��ͳ�͹

_____________					       || |

1 \ . R T . G / 5				       || |

6 \ . . . . / 9			2   3	RxD <��ٳ �

-------				3   2	TxD ����� �

5   7	GND �����ͼ

Note: You can use Null-modem cable as two-socket ComRing.

SOFTWARE

ComRing will not work with an ordinary SLIP or PPP packet driver,

you can't use it as null-modem. I wrote special packet driver to

make it usable. It is class 1 driver - it emulates DIX ethernet

(specification 1.11, basic and high-performance functions). You

must install it on each computer to close circuit.

To distinguish between different computers in the "network" you

must specify a single letter, one per computer. It is the first

parameter (and only required) to the packet driver. When you

invoke `comring a' on one computer you can't use `a' (or `A')

on others. This letter is used to generate ethernet address.

Syntax is:	comring id [parameter]...

You may use following parameters:

com#	    com port (default: 2)

baud=#   baud rate (default: 115200, it may be slowed down)

nodes=#  maximal number of computers connected (default: 3)

int=#    interrupt for packet driver (default: 60h)

If you're using nonstandard port you should specify:

port=#   port address

int=#    irq number

Example:  comring A baud=9600 com1

GAMES

Most games don't use packet driver interface. They expect IPX

network. You may use `pdipx' adapter. It must be configured to

use DIX ethernet frame:

econfig.exe pdipx.com shell:e 8137

Both programs are available in Simtel collection:

msdos/pktdrvr/pdipx103.zip, msdos/pktdrvr/novell.zip.

Please send me success/failure reports. (jb@inf.wsp.krakow.pl)

DISTRIBUTION

Distribution of this program is unlimited. You may sale it.

Remember that I don't guarantee that it works.