****************************************************************

*                                                              *

*          THE GUIDE TO HARD DISK SECTOR INTERLEAVING          *

*                                                              *

*                              by                              *

*                         Steven Gibson                        *

*                     GIBSON RESEARCH CORP.                    *

*                       Box 6024, Dept C                       *

*                       Irvine, CA, 92716                      *

*                        (714) 854-1520                        *

*                                                              *

****************************************************************

In these "sophisticated" days of computers, where AUTOEXEC.BAT

and CONFIG.SYS make casual conversation, it's rare to find a

topic as interesting and critical but still mis-understood, as

hard disk sector interleaving.

Our researches into this area have discovered that MOST IBM AND

COMPATIBLE personal computers are performance-crippled by mis-

interleaved hard disk drives.

Unfortunately, many disk controller companies, in competing with

each other, have set their disk interleave defaults too tightly

for many computers.  Such "specsmanship" directly hurts the

innocent computer user (you) by dramatically limiting his hard

disk data transfer rate.  This GUIDE carefully explains the

situation and shows how to use the two included programs to

determine whether your own IBM PC or compatible's hard disk

drives have their sector interleave set correctly.

Responding to this problem, Gibson Research Corp., publisher of

the popular display screen enhancement utility FlickerFree, has

recently developed an inexpensive software SOLUTION which first

determines your system's optimum hard disk interleave factor

then RESETS IT while leaving all your hard disk data in place!

UNDERSTANDING HARD DISK SECTOR INTERLEAVING

It's a rare person who would not wish for additional performance

from his personal computer's hard disk drive.  While much

attention is given to the drive's Average Seek Time, which is a

measure of the time required to move the read/write head from

one track to another, there is another vital detail which

determines overall hard disk performance and which is subject to

the user's control.

We will see that the too often neglected SECTOR INTERLEAVING

factor of a hard disk has a dramatic impact on data transfer

rates.

As most people know,  the information stored on a floppy or hard

disk is arranged in a series of concentric circular paths called

tracks.  The disk drive's read/write head is specifically

positioned over any desired track with an operation called a

SEEK.  Thus an obvious limit on the speed with which a drive can

find or place information would be the so called track-to-track

and average seek times.

A single track of a standard IBM compatible PC contains

approximately nine thousand bytes of data.  But since we usually

deal with data in much smaller chunks, each track is divided

into smaller sections called sectors.  Think of a spinning pizza

which has been cut into seventeen identical, and numbered,

slices.  (Drives with RLL encoding pack 50% more data onto every

track resulting in more than thirteen thousand bytes per track

divided into 25 or 26 sectors.)

Now suppose that we need to read the information contained in

sector 1 of our current track.  We patiently wait for sector 1

to rotate under our read/write head, reading its data at that

time.  After absorbing this freshly read information, we realize

that we also need to read the next sector, number 2.  However,

by the time sector 1 has been moved into our computer and we've

decided to read sector 2, the beginning of sector 2 has already

started passing under the read/write head.  So we have no choice

but to wait for the disk to rotate all the way around once more

to deliver sector 2.  If we wished to read a nine thousand byte

file composed of all seventeen disk sectors on this track,

seventeen complete rotations of the disk, one for each sector,

would be required!

It wasn't long before a bright engineer realized that the entire

problem could be easily resolved by spreading the sequentially

numbered sectors out around the disk:  Instead of placing sector

2 immediately after sector 1, sector 2 could be placed a few

sectors later!  In this way, after reading sector 1, sector 2

would be just about ready for reading by the time we were ready

for it.   Such an elegant solution!

If, for example, sequentially (logically) numbered sectors were

staggered out every three physical sectors, then each rotation

of the disk could read every third sector.  Therefore only three

revolutions of the disk would be required to read an entire

track.  Quite an improvement over 17 revolutions!  This sector

staggering is known as SECTOR INTERLEAVING or SECTOR MAPPING.

The physical spacing between logically consecutive sectors is

known as the INTERLEAVE FACTOR.  This example used an interleave

factor of three, shown as "3:1" and pronounced: "3 to 1".

The new higher-density RLL controllers need to be correctly

interleaved too.  With 26 sectors per track a non-interleaved or

mis-interleaved disk would require 26 revolutions for an entire

track transfer!

Now here's the real rub:  In the current environment of mix and

match highly modular personal computing, responsibility for

determining and setting your hard disk drive's optimal sector

interleave factor has "fallen through the cracks" as it were.

You've never worried about it have you?  If you're inclined to

believe that someone else has, (like your local dealer perhaps)

you might be in for a real surprise.  Experiments with a wide

variety of computers, drives, controllers, clock speeds, and

interleave factors have shown that the hard disks of MOST

PERSONAL COMPUTER SYSTEMS ARE NOT PROPERLY INTERLEAVED TODAY!

So many computers are so badly interleaved that it's quite

likely that you could increase your own hard disk's performance,

by FOUR TO SEVEN TIMES just by optimally setting your disk's

interleave factor!

The interleave factor can be either too "loose" or too "tight".

The result of operating with an interleave factor which is too

loose is lower performance than a particular drive-controller-

computer combination could achieve with tighter interleaving.

For example the original IBM PC/XT is interleaved at 6:1 but can

readily achieve 5:1 in a standard 4.77 Mhz machine and 4:1 in an

8 Mhz PC.  This means that disks on accelerated PC's can read

and write at 150% of their current rate!

The consequence of operating with an interleave factor which is

too tight is more disastrous, since missing that next sector

induces the significant delay of another entire disk revolution!

You might be saying to yourself "so what's an extra disk

revolution between friends?", but consider this:  If your disk

is properly interleaved, say with a factor of 3, the entire nine

thousand byte track will be read or written in just 3 disk

revolutions.  However, if the particular controller in your

particular computer cannot achieve that interleave of 3, then 17

or 18 entire revolutions will be required to read the same

track.

3 revolutions versus 18 is a performance difference of 600%!

Since many files and most programs are well over nine thousand

bytes long, and since DOS DOES REQUEST an entire track transfer

in such cases, these 17-revolution track reads add up quickly.

You would certainly feel the difference between waiting 3

seconds for a program to start up, versus 18 seconds!

The damning thing about proper hard disk sector interleave is

its total dependency upon EVERY factor of your system.  For

example, the AT&T 6300 requires 20 REVOLUTIONS for a full track

transfer when reading a disk which is interleaved at the most

common factor of 3:1!  Even it's faster brother, the 6300 Plus,

requires 14 revs!  We have found that the AT&T's hard disk data

transfer rate can be made FOUR TIMES FASTER when its interleave

is optimized for it!  If you're using a 6300 or 6300+, your disk

transfer rate is one quarter of what it could be!

Another example:  Western Digital's RLL hard disk controller

formats drives at a default interleave of 3.  Believe it or not,

this interleave is too tight for good old standard 4.77 Mhz XT!

Consequently 28 entire disk revolutions are required to read a

SINGLE track!  If the interleave is set to 4 then only 4 revs.

are required to read the same data, for a 700% throughput boost!

So by now you're dying to know what's happening with the hard

disk sector interleave of your own computers!  What interleave

factors are being used, are they correct, and what you can do

about it if not!

Included with this INTLEAVE.DOC file are two small programs:

SPINTEST.COM and SPINTIME.COM

The main program, SPINTEST.COM determines how many disk

revolutions your IBM compatible personal computer requires to

read an entire 17 sector track from its hard disk.  (SPINTEST is

also fully compatible with the 25 or 26 sector tracks used with

the high-density RLL controllers.)

Running SPINTEST on the many computers at Gibson Research, and

in many local computer dealerships, we discovered something

quite surprising:  Most computers being sold today are horribly

interleaved.  We turned up many machines which required more

than 17 entire revolutions just to read or write one full track!

These same machines flew along at 4:1 but not at 3:1.

For example, Gibson Research has a vintage Leading Edge Model-D

with a standard Western Digital hard disk controller.  This

machine always seemed to run more quickly than a newer generic

PC clone equipped with a different Western Digital controller.

Both hard disks received their original low-level (interleaving)

format at the Western Digital default interleave of 3.  After

all, those dealers must have thought, Western Digital knows

what's best.   Right?   NOT NECESSARILY!

Since the controller AND computer together determine the optimal

interleave, it is IMPOSSIBLE for any controller manufacturer to

set a single optimal interleave for everyone.  In fact, even the

controller's revision level can be a determiner!  One of our two

WD controllers ALWAYS MISSES sectors when the disk is inter-

leaved at 3, while the other controller makes 3:1 ... in the

same computer!

To test this, one of our hard disks was given a low-level format

at an interleave of four.  Afterward, the SPINTEST timing

diagnostic program and a stop watch confirmed our suspicions:

An MS-DOS file copy operation on the hard disk which had

required 139 seconds when the disk was interleaved at 3:1 now

took only 39 seconds at an interleave of 4:1!

Interestingly, that "slower" controller which can't make the

interleave of 3 on a standard 4.77 Mhz XT does just fine on a

faster 8 Mhz clone computer.  So as you can see, the COMPUTER

makes a difference too!  Any way you look at it, the issue of

sector interleaving involves the complex interactions among the

disk drive, the controller make and model (even the revision

level), the processor, and system clock rate.

OVERVIEW OF THE INTERLEAVE DIAGNOSTIC PROGRAMS

The issues of hard disk sector interleaving have gone unnoticed

for so long because there has never been a clear way to see

what's really going on deep inside a hard disk.  After seeing

the importance of this issue, we decided to change this.  The

two programs SPINTEST and SPINTIME determine, for any standard

MFM or RLL controller, exactly how many revolutions the disk

requires to transfer one entire track of data.  The programs

were hand written in 100% machine language (as is everything

Steve Gibson writes and Gibson Research publishes) to give them

the required measuring resolution speed.

THE SPINTEST PROGRAM

SPINTEST determines exactly how many revolutions your hard disk

currently requires to perform a full track transfer. DOS

transfers a full track whenever programs or files larger than 9K

are read or written ... which is most of the time.  SPINTEST

does NOT directly read your drive's interleave, but the drives

interleaving performance can be easily inferred from SPINTEST's

full track transfer revolution count.

SPINTEST averages the time required for each of over two hundred

full track reads in order to accurately determine the revolution

count per read, then the number of revolutions required to read

just one track and the controller's full-track data transfer

rate are computed.  SPINTEST only reads data from your drive so

data is NEVER altered.

Over two hundred track reads were used because of the

inconsistent disk transfer behavior of AT&T's 6300 machines.

These exhibit a maddening inconsistency in their ability to

transfer disk data.  Sometimes they get the next sector, and

often not.  This means that a much looser than normal interleave

generates MUCH BETTER overall disk throughput for AT&Ts.  For

this reason, and to guarantee correct results on any machines

which may behave similarly, SPINTEST performs many track reads

and averages the results.

THE SPINTIME PROGRAM

Since SPINTEST measures track read time, it must assume a given

ratio between elapsed time and disk rotation rate.  Some weirdo

clone computers have a clock which runs faster than normal when

in their "turbo" (8 Mhz) mode, and some lap computers have tiny

hard disks spinning at weird speeds.  If these do not sound like

your situation SPINTEST will deliver correct readings and you'll

not need SPINTIME's confirming measurements.

But if EITHER of these cases might be you (if you have a weirdo

clone computer or hard disk sporting laptop) SPINTIME will tell

you for sure.  SPINTEST is only usable when SPINTIME gives

standard readings.  The single (not surprising) exception to

this is for the AT&T 6300 and 6300 Plus machines.  Due the

overall problems they experience with disk transfers, SPINTIME

may show a lower-than-3600 RPM reading.  For 6300's this is

normal.

SPINTIME is interesting regardless since it determines EXACTLY

how fast your hard disk is really spinning!

SPINTEST

To run SPINTEST, simply type its name at the DOS prompt.

IMPORTANT!: Read the section IMPORTANT SPINTEST AND SPINTIME

NOTES below, before you begin!

Since SPINTEST takes the average of over 200 track reads, the

time required to run this test will vary between approximately

14 seconds for a fast 2:1 interleaved machine to 98 seconds for

an incorrectly interleaved 26-sector RLL encoded hard disk.  So

simply type: SPINTEST at the DOS prompt and wait a minute or

two.

SPINTIME

SPINTIME has two purposes:  To determine the speed of your

system's clock (if you suspect that it might be weird) , and to

verify that your system's hard disk is spinning at close to the

standard 3600 RPM standard.  Your system's clock is checked by

measuring SPINTIME's exact execution time.  It should require

EXACTLY 60 SECONDS to run ... no matter what.  During that time,

it is busy watching your hard disk spin, counting every revolu-

tion.  After EXACTLY 60 seconds, it displays your drive's exact

RPM (within its measuring resolution.)  IF SPINTIME's total

execution time is NOT EXACTLY 60 seconds, OR the displayed RPM

is not close to 3600, SPINTEST's reported revolution count won't

mean anything either.

SPINTIME's real value is to assure you that SPINTEST is

delivering worthwhile answers.

IMPORTANT SPINTEST AND SPINTIME NOTES!

Both programs assume and require DOS 2.0 or higher.  They

operate on your system's FIRST hard disk only (drive C).  The

hard disk must be bootable, have its controller plugged into a

slot, not requiring special CONFIG.SYS device drivers for

operation.

SPINTEST and SPINTIME will never disturb your hard disk data,

but if your disk is some kind of odd-ball they may not function

correctly.

The tests will be more accurate if your normal collection of

memory resident programs (if any) are not in memory at the time.

Many resident programs "steal" so much time from the computer

that a delicate timing analysis could be upset.  It would be

wise to boot your machine from a plain DOS diskette when running

these tests for maximum reliability.

Also, turn off (or better yet remove) any disk caching software

you might be using.  Disk caching intercepts the disk reading

performed by the programs and renders their measurements invalid

(and rather humorous).  By the way, disk caching program per-

formance is significantly improved through proper disk

interleaving.

WHAT IT ALL MEANS

So now you're wondering:  What do the numbers mean?  How do they

compare with industry norms and everyone else's?  Should I be

dancing on roof tops or wringing my dealer's neck?

A hard disk spins at 3600 revolutions per minute, or 60

revolutions per second.  A track has 17 sectors of 512 bytes per

sector.  This means that data passes under your drive's head at

a rate of 522,240 bytes per second.  (This number is 768,000 for

RLL con-trollers with 25-sector tracks.) Your system will

achieve some fraction of this maximum possible rate as

determined by the number of rotations required to read or write

each track.  522,240 (or 768,000) is divided by the number of

revs per track (as SPINTEST does) to calculate your drive's data

transfer rate.

Since your system's performance is meaningful only when compared

to other properly and improperly interleaved systems, the

following table will give you a feeling for where the industry

stands:

1:1 - There IS a controller which achieves 1:1,

unfortunately it lacks Error Correcting Code

(ECC) which is critical for reliable operation.

2:1 - Compac machines achieve 2:1.  Adaptec's 2070A

RLL controller can do 2:1 with an 8 Mhz machine

even though their low-level formatter stubbornly

refuses to allow a setting of less than 3!

3:1 - Newer WD controllers and many other controllers

will handle 3:1.  The Adaptec 2070A RLL con-

troller will do 3:1 in a 4.77 Mhz machine.

4:1 - Older WD controllers need 4:1, original IBM XT

controllers can do 4:1 in an 8 Mhz machine.

The AT&T 6300 Plus works best at 4:1 and WD's

RLL controller can only do 4:1.

5:1 - IBM's original XT controller can do 5:1 in a

4.77 Mhz XT but comes interleaved at 6:1.  The

AT&T 6300 is best at 5:1 which increases over-

all throughput 400% over its default!

6:1 - Only the original IBM PC/XT seems to have been

way out here.  These machines can be tightened

up, and if it has an inexpensive accelerator it

can probably run at 4:1 for 150% disk boost.

We have seen that WD controllers which miss at 3:1 will do

beautifully at 4:1.  Several InfoWorld readers have

independently confirmed that AT&T machines (with WD controllers)

perform best at the very loose interleave of 6:1.  Even the old

true blue IBM controller which runs at 6:1 can always do 5:1,

and even 4:1 in a faster than 4.77 Mhz machine.

DETERMINING FACTORS

So what system factors influence and determine the optimal

interleave setting for a given set of equipment?  After a sector

of data has been read it must be moved from the controller's on-

board buffer into the computer.  This is done with a process

known as DMA (Direct Memory Access.)  The time required to

transfer the sector determines how soon the controller will be

ready to read the next sector. For this reason add-in

Accelerator "Turbo" Cards do not generally change a machine's

optimum interleave since the main system clock speed, which

continues to control DMA memory accesses, is not changed.  The

less expensive "Speed Booster" products which alter the basic

system clock timing DO have a tremendous impact on optimal

interleave by running the DMA faster and thus moving the data in

and out faster.

GIBSON RESEARCH AND INTERLEAVING

When we began these experiments we had NO IDEA that so many

personal computers were so poorly interleaved.  Without the aid

of our new special software, changing a hard disk's sector

interleave "manually" requires first backing-up all hard disk

data onto some secure medium.  Then a low-level re-formatting is

performed.  This unfortunately messy task was never designed for

the casual computer user since it requires unpublished knowledge

of the internal details of your controller, using the DOS DEBUG

command to poke hexadecimal values into the 8088's machine

registers and starting the low-level format.  After this, the

FDISK and FORMAT commands are used to create a partition table

and lay down the high-level formatting information.  After all

this, the backed up data must be copied back onto the drive.

Then, if the experimentally chosen interleave was not correct,

most of the process would have to be repeated with a different

trial interleave factor.

Gibson Research Corp., responding to the clear need for a better

solution to the task of hard disk sector interleaving optimiza-

tion has automated and streamlined this "re-interleaving"

process with a product which was designed specifically to meet

this need.

This software product quickly determines the optimal interleave

setting for any disk of any size in any system with any clock

rate.  Once determined, the existing interleave of the disk can

be RESET automatically, and in just a few minutes, leaving all

your disk data intact and in place!  The product also performs

several other useful hard disk utility functions which have

never before been available.

GET YOUR DISK SPINNING RIGHT!!

Even if SPINTEST turned in excellent results of 3 or 4 revs, you

should STILL check out Gibson Research's new product for the

other surprising (NEVER before offered) hard disk capabilities

it brings ... and if you received results like 10, 12, or even

17, 18, or 19 revolutions, there is NO DOUBT that your system

could be running FOUR TO FIVE TIMES FASTER ... in minutes!

To receive all the details about this product simply phone, or

mail your name and address.  Ask for the INTERLEAVING INFO from

the On-Line edition of the GUIDE. ... you'll be glad you did!!

GIBSON RESEARCH CORP.

Box 6024, Dept C

Irvine, CA 92716

(714) 854-1520

(If you're wondering about Gibson Research's first

product, FlickerFree, ask for some info about it too!)

****************************************************************

*                                                              *

*               <-- The End ... Of the GUIDE -->               *

*                                                              *

****************************************************************