PC-HAM WHATSON.DOC Version 2.0 PAGE 1

PC-HAM Version 2.0

G3ZCZ AMATEUR RADIO CONTEST PACKAGE VERSION 2.0

(C) Copyright 1985, 1986 - All Rights Reserved

By Joe Kasser G3ZCZ

POB 3419,

Silver Spring,

Md., 20901, USA.

PC-HAM  is a set of DATA Processing Amateur Radio  programs  for

the user of a microcomputer in Amateur Radio. The BASIC language

programs  may or may not be described in or be based on programs

described in the book, SOFTWARE FOR AMATEUR RADIO written by Joe

Kasser,  G3ZCZ, published by TAB Books (Number 1560), Blue Ridge

Summit,   Pa.,  17214.  The  DBASE2  programs  perform  database

operations on logbook information.

PC-HAM is written and supported by Joe Kasser, G3ZCZ.

The  program  is  distributed as a Shareware  product.  You  may

freely copy and share the product with your friends,  associates

and other radio hams.  If you decide to use the product, you are

asked  to become a registered user by sending  a QSL card and  a

contribution   to  the author (suggested amount of  $36.50  (ie.

half of 73) or equivalent in foreign currency (See  registration

form below).

The  contribution covers the cost of distribution of the updated

disk,  and  supports the continual development of "Software  for

Amateur  Radio".   Upon receipt of your contribution,  you  will

receive  one free update disk ),  mail (electronic and  regular)

support,  and notice of further releases.   The update disk will

contain any new versions of the software on this disk (converted

programs  from  Northstar  BASIC  to  IBM)  plus  new  software,

accompanying  the series of articles currently being written  on

the  applications of data processing concepts of  microcomputers

to amateur radio.

Under  no circumstances may this product be sold or  distributed

with  another product without the express written permission  of

Joe Kasser, G3ZCZ.

Joe  Kasser,  G3ZCZ will only support unmodified copies  of  this

software.  Your comments and suggestions for changes are however

welcome.   If  you  are  the first to suggest a change  that  is

implemented,  you will be sent a complimentary copy of the  disk

with the change incorporated.

PC-HAM WHATSON.DOC Version 2.0 PAGE 2

TABLE OF CONTENTS

1.0 INTRODUCTION

2.0  STARTING THE PROGRAM

3.0  RUNNING THE PROGRAM

3.1  Date for Predictions.

3.2  Sunspot Number.

3.3  Output Routing

3.4  Mode Selection

3.4.1     Single PX

3.4.2     Contest Sheet

3.5  Prefix Loading

4.0  NOTES

1.0  INTRODUCTION

WHATSON  is a program that can be used to provide predictions of

HF  propagation.   It  is  based on a Minimuf devised  by  P  .H

Levine, R. B. Rose, and J. N. Martin and then modified by G3CCZ,

G4LH, G4GKO, 4X4AS and G3ZCZ

The  program  is  based on a  particular  model.   The  original

program   was  published  in  the  March  1983  issue  of  Radio

Communication  (published by the Radio Society of Great  Britain

(RSGB)).   A number of events can affect actual propagation,  so

use the model with foresight.

2.0  STARTING THE PROGRAM

The program is loaded and run in the general manner for  loading

and  running  programs  written in BASIC.   Thus BASIC  must  be

loaded  first and then the program is loaded.   Once  the  "RUN"

instruction  has been given to the computer,  it will ask you  a

number of questions to determine exactly what you want it to do.

3.0  RUNNING THE PROGRAM

3.1  Date for Predictions.

You will be asked to tell the computer what date the predictions

are for, by the following question.

PC-HAM WHATSON.DOC Version 2.0 PAGE 3

Are predictions for today (Y/N)

If you want the predictions for that date, the program will read

the  system  calendar.   If you want predictions for some  other

day, you will be asked to enter that date.

3.2  Sunspot Number.

You  will be asked to tell the program what the  Sunspot  number

for that date is.  If you don't know, try getting it from WWV or

the predictions in your favorite Ham magazine.

3.3  Output Routing

You may tell the program to print the output, or just display it

on the CRT screen.

3.4  Mode Selection

The program operates in two modes.

3.4.1     Single PX

The  single  prefix  mode,  gives  you  a  display  of  the

propagation  forecast for one call area for one  particular

day.

A  typical display/printout starts off with a header  which

contains  the  date and sunspot numbers.   You QTH and  QRA

Square  locator information are then  given.   The  program

next  shows  the  path,  target  area  prefix  bearing  and

distance of the target.  The last item on the header is the

date  and  time that the printout/display was  made,  again

taken from the system clock/calendar.

The  propagation  forecast is given for every hour  on  the

hour.   The Highest Possible Frequency (HPF),  the  Optimum

Usable  Frequency  (FOT)  and the Lowest  Usable  Frequency

(LUF)  are  then  displayed in a tabular  form.   When  the

display  is  complete,  you are asked if you want  to  have

another go.

If  no propagation is possible at a  particular  hour,  the

display will show dashes instead of a number.

PC-HAM WHATSON.DOC Version 2.0 PAGE 4

----------------------------------------------------------------

DAY: 19   MONTH: 7   SUN SPOT NUMBER: 0

G3ZCZ/4X  LOCAL QTH IS:LAT. 31.895 N/LON. 34.791  = KM71JV

SHORT PATH TO K2, BEARING: 314 deg.  DISTANCE: 9095  KM

DATE & TIME OF PRINTOUT:07-19-1986/23:12:37

GMT   HPF    FOT    LUF               GMT   HPF    FOT    LUF

0    14     14      5                 1     ------------

2     ------------                    3     ------------

4     ------------                    5     ------------

6     ------------                    7     ------------

8     ------------                    9    14     14     12

10    15     15     12                11    16     15     12

12    17     16     12                13    18     17     11

14    19     17     11                15    19     17     11

16    20     18     11                17    20     18     11

18    20     18     11                19    19     18     10

20    20     18      9                21    19     17      8

22    17     16      7                23    15     15      5

DO YOU WANT ANOTHER QTH Y/N ?

----------------------------------------------------------------

3.4.2     Contest Sheet

The  Contest  sheet  mode  gives  you  a  printout  of  the

direction and distance as well as the optimum frequency for

propagation  every  hour  on the  hour,  for  every  prefix

available.   It  is designed for use when planning  contest

operation,  for with it you can see what the most favorable

time for predicted openings to different parts of the world

are, and plan band changes (or rest periods) accordingly.

Take  a look at the closest bands to the optimal  frequency

and see what gives.

A typical display contains the following data.

PC-HAM WHATSON.DOC Version 2.0 PAGE 5

----------------------------------------------------------------

SHORT PATH OPTIMAL FREQUENCY PREDICTIONS FOR G3ZCZ/4X ON  19 Jul

SUNSPOT NUMBER  0

PX      1  2  3  4  5  6  7  8  9 10 11 12 13 14 15 16 17 18 19

----------------------------------------------------------------

FG7    15 -- -- -- 14 14 -- -- -- -- 16 18 19 20 19 19 19 19 19

FH8    -- -- -- -- -- 15 17 18 18 19 19 18 18 17 17 17 17 17 17

FK8    -- -- 16 18 20 20 20 19 18 23 22 20 19 17 16 15 14 -- --

FM7    15 -- -- -- 14 14 -- -- -- 14 16 18 20 20 19 19 19 19 19

FO8    -- -- 15 17 18 18 18 17 16 15 -- -- -- -- -- -- -- 14 16

FO8Clp 15 -- -- -- 15 16 17 16 15 -- -- -- -- 16 18 19 20 19 19

FP8    15 -- -- -- -- -- 14 16 16 17 18 18 19 19 19 19 19 19 19

FR7    -- -- -- -- 16 18 20 21 22 22 21 20 19 18 18 18 18 18 19

FS7    15 -- -- -- 14 14 -- -- -- -- 16 17 19 20 19 19 19 19 19

FW8    15 -- -- -- -- -- -- -- -- 15 18 20 21 20 19 19 19 19 18

FY7    15 -- -- -- -- -- -- -- -- 15 18 20 21 20 19 19 19 19 18

G      15 -- -- -- 15 16 17 18 19 19 20 20 20 20 20 20 20 20 19

GD     15 -- -- -- 15 16 17 18 18 19 19 20 20 20 20 20 20 20 20

GI     -- -- -- -- -- -- -- 14 15 15 15 16 16 16 16 16 16 15 15

GJ     15 -- -- -- 15 16 17 18 19 20 20 20 20 20 20 20 20 20 19

GM     -- -- -- -- -- -- -- 14 15 15 15 15 16 16 16 16 15 15 15

GU     15 -- -- -- 15 16 17 18 19 20 20 20 20 20 20 20 20 20 19

---------------------------------------------------------------

3.5  Prefix Loading

The computer will then load the local station geographical  data

from  the file G3ZCZ.DAT,  and then the data associated with all

the remaining amateur prefixes WHATSON.DAT.   It will display  a

message as follows while that job is going on.

PLEASE STANDBY WHILE PREFIX DATA ARE LOADED

If  you  elected  the Contest mode,  it will begin  without  any

further ado.   In the single prefix mode,  you will be presented

with the following menu.

PC-HAM WHATSON.DOC Version 2.0 PAGE 6

----------------------------------------------------------------

PREFIX MENU

NBR   PX            NBR   PX            NBR   PX            NBR   PX

1   - 1A0KM         18  - 4T            35  - 5X            52  - 8Q

2   - 1S            19  - 4U1ITU        36  - 5Z            53  - 8R

3   - 3A            20  - 4U1UN         37  - 6C            54  - 8Z

4   - 3B6           21  - 4V            38  - 6D            55  - 9G

5   - 3B8           22  - 4W            39  - 6O            56  - 9H

6   - 3B9           23  - 4X            40  - 6T            57  - 9J

7   - 3C            24  - 4Z            41  - 6W            58  - 9K

8   - 3C0           25  - 5A            42  - 6Y            59  - 9L

9   - 3D2           26  - 5B4           43  - 7J            60  - 9M2

10  - 3D6           27  - 5H            44  - 7O            61  - 9M6

11  - 3V8           28  - 5L            45  - 7P            62  - 9M8

12  - 3X            29  - 5N            46  - 7Q            63  - 9N

13  - 3Y            30  - 5R            47  - 7X            64  - 9Q

14  - 4D            31  - 5T            48  - 7Z            65  - 9U

15  - 4J1           32  - 5U            49  - 8J1           66  - 9V

16  - 4N            33  - 5V            50  - 8N            67  - 9X

17  - 4S            34  - 5W            51  - 8P            68  - 9Y

Pick a number corresponding to a prefix (0 will give you further choices )?

---------------------------------------------------------------------------

If  you  enter  the number 0,  you will get a  further  menu  of

choices.   If you enter 999,  you will be asked to redefine your

local geographical co-ordinates.   You must run this choice  the

first  time  or  the data you generate will  have  no  practical

purposes for your QTH.

4.0  NOTES

The program is based on a mathematical model.  In comparing logs

with  the predictions,  it has been noted that the data tends to

lose  accuracy when the path length is greater than 8000  kM  or

the  path  passes  over  the geographical  poles.   If  you  are

interested enough in the further development of the  model,  why

not  compare  your logs against the predictions.  You should  be

able  to get hold of sunspot records from magazine  back  issues

and let me know how they compare.

#################################################################################################