WESTERN DIGITAL

"PLUS"-FAMILY LAN ADAPTER BOARD DRIVERS

FOR USE WITH NOVELL ADVANCED NetWare AND SFT NetWare

v2.1 AND v2.11

INSTALLATION GUIDE

This directory provides drivers for the "PLUS" (WD8003) family of

Western Digital LAN adapter boards:

DRIVERS         WESTERN DIGITAL ADAPTER BOARD TYPES

WD8003            StarCard PLUS (WD8003S)

StarLink PLUS (WD8003SH)

EtherCard PLUS  (WD8003E, WD8003EBT)

Included on this directory are the following files:

INSTALL.DOC                  This File.

RELEASE.DOC                  Version number and specific

release information.

WD8003.LAN                   The LAN driver descriptor file.

WD8003AD.OBJ                 The Server A-Driver object file.

WD8003BD.OBJ                 The Server B-Driver object file.

WD8003CD.OBJ                 The Server C-Driver object file.

WD8003DD.OBJ                 The Server D-Driver object file.

WD8003SH.OBJ                 The Shell Driver object file.

To install the drivers on the Novell Advanced NetWare v2.1

installation diskettes, copy the following files onto the

specified Novell diskettes:

WD FILE      Copy To    Novell Diskette

-------                 ---------------

WD8003.LAN -----------> AUXGEN & SHGEN-1

WD8003AD.OBJ\

WD8003BD.OBJ \

WD8003CD.OBJ  >-------> LAN_DRV_002

WD8003DD.OBJ /

WD8003SH.OBJ/

Once the drivers are installed on the Advanced NetWare diskettes,

proceed with the normal NETGEN and SHGEN procedures.  At the

point in the procedures where you are directed to select the

LAN drivers, the WD8003 drivers will appear in the list of

choices as follows:

WD Star/EtherCard PLUS

There are various configurations of the drivers that you can

select.  Some are designated for adapter boards with 8 Kbyte RAM

buffers, and some for boards with 32 Kbyte RAM buffers.  The

drivers configured for 32 Kbyte RAM buffers will automatically

revert to using 8 Kbyte RAM if they find that the adapter board

only contains 8 Kbyte RAM or is configured to only use 8 Kbyte RAM.