COMMIX NETWORK SOFTWARE CABLE/MODEM VERSION FOR MS-DOS

2 OCTOBER 1987

ITRON

A DIVISION OF INFOTRON

SUITE 116

130 GAITHER DRIVE

MOUNT LAUREL, NEW JERSEY 08054

(800)423-8044

(609)722-5575

INDEX

1.   INTRODUCTION

General Description

CNS Functions

Files

Manual Outline

2.   INSTALLING CNS ON YOUR PC

Copying CNS to Your PC Boot Disk

Modifying Autoexec.bat

COMMIX Network Software Menu Structure

Setting up a Modem

Cabling PC's Back to Back

3.   CONFIGURING CNS FOR YOUR NETWORK

Configuration Sub-Menu

Printer Setup

File Transfer Method

File Service Access

Serial Port Configuration

Terminal Emulation Destinations

Save Configuration

4.   USING COMMIX NETWORK SOFTWARE

Introduction

File Transfer

File Service

Printer Control

Terminal Emulation

5.   TROUBLESHOOTING

Hayes is a trademark of Hayes Microcomputer Products Inc.

VT100 is a trademark of of the Digital Equipment Corporation.

COMMIX,  COMMIX  32  and  COMMIX  Network  Software  are  trademarks of

Infotron Systems Corporation.

Lotus 123 is a trademark of the Lotus Development Corporation

WordPerfect is a trademark of the WordPerfect Corporation

IBM and IBM PC  are trademarks  of the  International Business Machines

Corporation.

MS-DOS is a trademark of the Microsoft Corporation.

Leading Edge is a trademark of Leading Edge Hardware Products, Inc.

Sidekick is a trademark of Borland International,

SECTION 1

INTRODUCTION

GENERAL DESCRIPTION

COMMIX Network Software (CNS) permanently resides in the memory of a PC

and provides seamless and transparent COMMIX and Hayes-compatible modem

functionality to  PC users.  CNS transfers files, manages printers, and

performs terminal  emulation. CNS  is normally  used with  one of three

hardware configurations:

1)   HARDWIRE. Two PC's are directly cabled together through their

serial communications  ports. A  printer may  be connected to

the PC  via a  serial connection. CNS allows software written

to communicate with parallel  printers  to  work  with serial

printers using XON/XOFF flow control.

2)   MODEM. A  PC communicates  with another  PC, printer, or host

computer via a modem connection.

3)   COMMIX 32. Several PC's, modems, printers, and host ports are

cabled to COMMIX 32 ports. (The file READ.ME on this diskette

will tell you more about the COMMIX 32.)

In a  COMMIX  network,  CNS  automatically  makes  and breaks

connections through  COMMIX to provide these services. COMMIX

Network Software works across  COMMIX Ethernet  Links as well

as within a single COMMIX node.

COMMIX Network  Software resides  in the  background memory of a PC and

will not interfere with most other PC application software. A  user can

be working  with Lotus 123(c) or WordPerfect(c) at the same time CNS is

transferring a database.

COMMIX Network Software is controlled and  set  up  via  a  pop-up menu

system. The  menus can  be accessed at any time by pressing the ALT and

period key at the  same  time.  This  is  known  as  a  "hot  key." For

example,  a  user  in  the  middle  of working on a spreadsheet, simply

presses ALT-period and the  menu appears.  When the  user is  done, the

screen and application are completely restored.

As  menu  options  are  chosen,  new  menus  overlay previous ones. The

ESCape key moves to previous  menus.  Pressing  ESCape  at  the initial

menu returns to an application or DOS prompt.

CNS FUNCTIONS

FILE TRANSFER  -- user selects a system from the pop-up menu and a file

to send  or receive  from that  system. CNS  makes the connection,

transfers  the  file,  then  breaks  the  connection. The transfer

happens in background so the user can work  on other  tasks during

1-1

the transfer. The File Server capabilities allow a user to do file

transfers with no user intervention on the remote system.

FILE SERVICE -- sets up your PC to permit easy file transfer  access to

other users.  Several directories  can be set up with combinations

of READ, CREATE, OVERWRITE,  or  NO  access.  The  actual transfer

itself is initiated by a user on the network via the File Transfer

options of CNS. CNS is a two-way street; it can both initiate file

transfers and respond to transfer requests from a distant machine.

PRINTING --  redirects all  print jobs from a parallel port to a serial

port. If required, CNS can make a connection  through COMMIX  to a

printer or  dial a  call to  a printer  through a modem (the modem

must be configured for  auto-answer). It  initializes the printer,

prints the job, then breaks the connection. The pop-up menu allows

the user to select  from six  different printers,  each with their

own set of configuration parameters.

If less  than six  printers are used, the same printer can be used

as  several  logical  printers,  each   with   its   own   set  of

characteristics. For example, printers DRAFT and LETTER can be the

same  dot  matrix  printer.  When  printer  DRAFT  is  chosen, the

initialization string  causes the  printer to  work in draft print

mode, printer LETTER tells it  to  print  in  near  letter quality

mode.

TERMINAL  EMULATION  --  pops  up  an  ASCII terminal and automatically

makes a connection through COMMIX or dials a call through a modem.

Four predefined  destinations can  be chosen.  User can also start

terminal emulation without making a connection or a call.

FILES

There are four files on this diskette:

CNSMC.COM --   The COMMIX Network Software program.

CNS.CFG --     Configuration file for CNS.

CNS.DOC --     The users manual for CNS

READ.ME --     An quick introduction to CNS and this diskette,  as well

as an  introduction to  ITRON's COMMIX 32 office network

system.

Section two of this manual will tell you how to install CNS on your PC.

1-2

MANUAL ADDENDUM OUTLINE

This manual addendum is divided into five parts:

INTRODUCTION

INSTALLING CNS ON YOUR PC

CONFIGURING CNS FOR YOUR NETWORK

USING CNS consisting of information on

File Transfer

File Service

Printer Control

Terminal Emulation

TROUBLESHOOTING

1-3

SECTION 2

INSTALLING CNS ON YOUR PC

There are four phases of installing COMMIX Network Software on your PC:

1)   Copying the  software from the ITRON distribution diskette to your

hard disk, or the floppy from which you boot your system.

2)   Placing commands in your AUTOEXEC.BAT  file  which  places  CNS in

memory whenever you boot your system.

3)   Learning the CNS menu structure.

COPYING CNS TO YOUR PC BOOT DISK

To copy  the files  on to your hard drive or floppy from which you boot

your PC, place the ITRON CNS  distribution disk  in one  of your floppy

disk drives.  Then set your default directory to the directory you wish

the CNS  files to  reside. It  doesn't matter  where you  place the CNS

files, so  put them in a directory which suits your needs. Finally, use

the DOS COPY command  to copy  the files  named CNS.COM  and CNS.CFG to

the hard drive or boot disk.

For example, to install CNS in a directory called \SYS\COMMAND on drive

C:, place  the  ITRON  distribution  disk  in  drive  A:  and  type the

following commands at the DOS prompt:

C:

MD \SYS\COMMAND     (unless directory already exists)

CD \SYS\COMMAND

COPY CNS.*

MODIFYING AUTOEXEC.BAT

Once the  CNS files  have been placed on your boot disk, you should add

the following commands to a file called AUTOEXEC.BAT:

CD \SYS\COMMAND

CNS

PATH \SYS\COMMAND

These commands  will  place  COMMIX  Network  Software  into  your PC's

memory whenever you start your system.

Once installed,  COMMIX Network  Software is ready to serve you. Simply

press the ALT-period "hot key" to activate the pop-up menu.

2-1

COMMIX NETWORK SOFTWARE MENU STRUCTURE

The main menu shows several options.  To select  an option,  use the up

and down arrows to move to highlight bar to the desired option. You may

also type the first letter of the option to select it. Once  the option

is selected,  press the  RETURN or  ENTER key  to activate it. The FILE

SERVICE option  allows you  to enable  or disable  a feature. Selecting

the other  features will  take you into sub-menus. Sub-menu options are

selected and chosen in the same manner as they are in the main menu.

To leave a sub-menu and return to a higher level menu, press the ESCape

key. To  return to  DOS or  your application  from the main menu, press

ESCape. If you select  an option  by mistake,  simply press  ESCape and

you'll go  back to  the menu  from which  you selected  the option. The

COMMIX Network Software menus and their functions are discussed in more

detail in the next two sections of this manual.

DOS AND MACHINE COMPATIBILITY

The CNS  software should  operate correctly  with all 99% IBM PC clones

running PC-DOS or  MS-DOS  version  2.0  or  later.  The  software will

automatically support  monochrome, Hercules,  CGA, EGA and VGA displays

while in text mode.

The software is known to operate correctly on the following machines:

IBM PC:        XT, AT, ATX

IBM PS/2:      30, 50, 60

Amstrad:       1512, 1640 (mouse driver not installed)

Zenith:        Z-181, Z-183

Leading Edge:  Model D

The CNS software will not run  correctly if  the machines  system clock

"tick" is  not 18.2 per second. For example CNS will not work correctly

with the Amstrad range of machines if the mouse driver is installed.

The software  will  NOT  normally  operate  with  the  "DOS compatible"

operating  systems  because  the  program  may be swapped out of memory

while communicating in the background, or the screen may be in graphics

mode. (For  the same  reasons CNS should NEVER be swapped out of memory

using some TSR program selectors as CNS may have a file open and serial

data being transmitted).

Systems to avoid include:

PC-MOS 386

DosMerge 386

Microsoft Windows (1.x or 2.x)

OS/2.

2-2

Alternate versions  of CNS  will be  available to support some of these

other operating systems in the future.

INTERACTION WITH OTHER PROGRAMS

The CNS software occupies less  than  40K  of  memory  when  loaded and

provides all  the functions  outlined in  section 1.  The whole package

must be loaded, it is not possible to selectively load functions.

During file transfers the CHKDSK command  must be  avoided. The program

assumes that  no files  are currently  being accessed,  and will report

any partially written file from CNS an error.

* NEVER RUN CHKDSK/F WITH CNS ACTIVE. *

If a file is "typed" and the scroll  lock or  CONTROL-S entered  in the

middle of  the file,  then any  current file  transfer may  fail due to

timeouts. This will only  occur if  the screen  is held  stationary for

more than 60 seconds.

The CNS  software may  be loaded  with Borland's Sidekick. Sidekick, as

usual, must be loaded last.

The CNS software should  coexist with  most other  popup software which

does not access the selected serial port.

SETTING UP A MODEM

If you  are using  COMMIX Network Software with a modem, the modem must

be configured to hang up the call when your PC  drops DTR.  You'll need

to  consult  your  modem  documentation  to  see how this is done. Some

modems (such as the Hayes 1200) require you to set  hardware switchs on

the modem,  others (like  the Hayes 2400) allow you to set this through

software.

On a Hayes 2400 or full compatible you would use CNS to  enter terminal

emulation without  dialing a  call (see section 4 for details). You can

now enter "AT" commands directly to the modem. Then type:

AT&D2&W <RETURN>

This will set the modem. Then  press FUNCTION  KEY 9  to leave terminal

emulation.

CABLING PC'S BACK TO BACK

If you  are connecting two PC's back to back you must use what is known

as  a  "cross-over"  cable.   This  can   be  purchased   from  a  data

2-3

communications equipment  distributor or computer store. If you wish to

make your own cable see the use the following instructions:

DB-25 CONNECTOR

Pin  3    to pin2

Pin  2    to pin3

Pin  7    to pin7

Pin  6    to pin20

Pin  20   to pin6

DB-9 CONNECTOR

Pin  2    to pin3

Pin  3    to pin2

Pin  5    to pin5

Pin  1    to pin6

Pin  6    to pin1

2-4

SECTION 3

CONFIGURING COMMIX NETWORK SOFTWARE FOR YOUR NETWORK

Once you have installed CNS software on your PC, you'll  need to tailor

CNS to  your particular network. In many instances the default settings

will be sufficient for  your  network.  You  can  make  a  standard CNS

configuration and  distribute it to your PC users, or you can configure

CNS separately on each PC.

There are seven steps to properly configuring COMMIX Network Software:

1)   Enter the  configuration  sub-menu  of  CNS  and  perform the

following seven steps.

2)   Set  up  printer  name,  telephone number or COMMIX port, and

initialization strings for each printer.

3)   Define file transfer method.

4)   Define file service access.

5)   Set serial port number and speed.

6)   Define four predefined terminal emulation destinations.

7)   Save all  configuration settings  to a  configuration file so

that the  settings will  be active  when you turn on or reset

your PC.

CONFIGURATION SUB-MENU

Once you have drawn your  network,  you  are  ready  to  begin software

configuration. Press ALT-period to bring up the COMMIX Network Software

menu. Use the arrow key, or  press "C"  to select  configuration. Press

RETURN and you'll see the configuration menu.

Once in  the Configuration sub-menu, use the arrow keys or first letter

of the option to select the option. Press  RETURN to  go into  the sub-

menu for that particular configuration option.

Once  you  are  done  configuring  select  SAVE  CONFIGURATION from the

configuration  sub-menu  to  save  all  configuration   settings  to  a

configuration file so that the settings will be active when you turn on

or reset your PC.

PRINTER SETUP

Use the arrow key,  or press  "P" to  select PRINTER.  Press RETURN and

you'll  see  the  printer  configuration.  For each of the six possible

printers, there are three options to be set:

1)   The printer name which appears in the printer sub-menu.

2)   The printer destination. If the printer is cabled directly to

your PC,  leave this blank. If you are communicating with the

3-1

printer through a modem, this should be the  telephone number

of the  modem to  which the  printer is  attached. If you are

networking through a COMMIX 32,  this  should  be  the COMMIX

port number or name to which the printer is attached.

3)   The initialization  string to be sent to the printer whenever

a print job is sent. Consult the documentation  supplied with

the printer  to see  what these  codes are, as they vary with

printer. Control characters  can  be  included  by  using the

tilde (^) character before a letter. For example control-O is

represented as ^O. The escape character is ^[.

One physical printer can serve as several CNS printers. For  example, a

Star(c) printer  could be  cabled to COMMIX port 12 with a port name of

"STAR" . You could then configure two CNS printers, "STAR  DRAFT" which

prints  on  the  Star  printer  in  draft mode, and "STAR LETTER" which

prints in letter quality mode. Both CNS printers would send  print jobs

to  the  same  COMMIX  port,  but  would  send different initialization

strings to the printer.

To select a printer to configure  enter the  number of  the printer, or

use the  arrow to  move the  highlight bar. Press RETURN and you'll see

the configuration options for that printer. Select an  option by typing

its first  letter or  using the  arrows, then  press enter. You can now

type the printer name shown on the menu,  the printer  destination, and

the  initialization  string.  Press  RETURN  when  finished typing each

option.

Besides setting  up six  printers, you  can also  write a  banner to be

printed with  each printout.  A good  idea would  be to have the banner

different for each PC so that  printouts can  be easily  distributed to

the requesters.  Type "B" or use the arrow to move the highlight to the

banner option.  Press RETURN  and type  the banner  followed by another

RETURN. If no banner is specified, then the banner page is not printed.

FILE TRANSFER METHOD

COMMIX Network  Software allows  you to  exchange files with another PC

using CNS, or with another system (including PC) using the  Kermit file

transfer protocol. You will need to configure CNS accordingly.

Selecting FILE  TRANSFER OPTION  from the configuration menu allows you

to set to options:

1)   Whether to use seven or eight bit bytes during file transfer.

If  you  are  using  a  modem  which  only  passes  seven bit

characters, choose seven bits; otherwise, use eight bits.

2)   Default directory for file transfers. This  is only important

if Kermit  is calling  CNS. Since  Kermit can  not change the

3-2

remote directory, you must pre-set it in CNS.

With CNS to  CNS  file  transfers,  you  can  set  the remote

subdirectory with local CNS.

Use of  Kermit programs  and terminal  emulations not  written by ITRON

with the CNS file service is not supported and is done at  the your own

risk.

Select FILE  TRANSFER OPTION  SELECTION from  the Configuration menu to

change these settings. To  change  the  number  of  bits  select HI-BIT

CONTROL and press RETURN to toggle between seven and eight bits.

To  change  the  default  directory  select  DEFAULT DIRECTORY from the

Configuration menu, press  RETURN,  type  the  default  directory, then

press RETURN.

FILE SERVICE ACCESS

File Service  Access Control allows you set file transfer access to DOS

subdirectories on your PC. This controls the access  of users  on other

PCs. COMMIX  Network Software  will not  interfere with  your access to

files on your PC.

You can set access  for up  to seven  subdirectories. You  can also set

default access  for the remaining subdirectories not specifically given

access.  A  password  prevents  unauthorized  people   from  seeing  or

modifying your file service access settings.

Four levels of access can be granted:

1)   READ ONLY  -- remote  users may  only receive files from that

subdirectory. They cannot send to it.

2)   WRITE ONLY -- users can send  files to  the subdirectory, but

cannot receive for it.

3)   READ/WRITE --  users can both send and receive files from the

subdirectory.

4)   NO ACCESS -- users  can neither  send nor  receive files from

the subdirectory.

The configuration  options for file service access is split between two

sub-menus within the configuration menu. Menu one sets  access for four

subdirectories; menu  two sets  access for  three menus, as well as the

default access and the password for using the File Service  Access sub-

menus.

To  view  or  modify  file  service  access  select File Service Access

Control 1 or 2 from the configuration sub-menu. After you  press RETURN

3-3

you  will  be  asked  for  a  password.  The  initial password on a new

installation of CNS is "ITRON." You  may  change  the  password  in the

File Service Access Control 2 menu. After entering the correct password

you'll see File Service Access Control menu.

To add or modify a subdirectory, select a directory, press RETURN, type

the subdirectory name, then press RETURN. To remove a subdirectory from

the list, select it, press RETURN, and press the space  bar followed by

another RETURN.

To modify  access, select  the access  option for the subdirectory, and

press RETURN until the access you desire is displayed.

The password for the File Service Access Control menus can  be modified

in  the  File  Service  Access  Control  2 menu. Select Password, press

RETURN, type the new password, and again press RETURN.

Since the access list is always scanned from  entry 1  to entry  7, and

the scanning  stops at  the first  match, a  very powerful file control

system can be setup.

For example if a user wished  to exclude  access to  all of  drive 'C:'

except for a directory "\external" where users may read files, and

a directory  "\external\write" where  users might read and write files,

the following setup would be used:

C:\external\write            Read/write access

C:\external                  Read only

C:                           No access

The order is important. If the entry for "C:" was first then  the whole

disk would be protected.

SERIAL PORT CONFIGURATION

The  serial  communications  port,  speed  and  dialing  used by COMMIX

Network Software is set in the  COM PORT  SELECTION AND  SPEED sub-menu

of the Configuration menu.

Select  COM  Port  used,  and  press RETURN to toggle between COM1: and

COM2: of your PC. If you  do not  know which  COM: port  to use, you'll

need to  consult with  the person who sold you your PC or installed the

hardware. If your PC has one serial port it is most likely COM1:.

Select Baud Rate Used, and press RETURN to choose a  speed between 1200

and 19,200 bps.

The dialing  method allows  you to  tailor CNS to the type of telephone

service your modem uses. The following options are available:

1)   Touch-tone for telephone circuits with that capability.

3-4

2)   Pulse Dialing for those without touch-tone capability.

3)   COMMIX port name. This is used  when your  PC is  cabled to a

COMMIX 32 unit rather than a modem or another PC.

4)   Disabled when you don't wish CNS to place calls.

To choose  a dialing  method, select DIALING and press RETURN until the

desired setting is displayed.

TERMINAL EMULATION DESTINATIONS

The Terminal Emulation Destinations and Terminal  Type sub-menu  of the

Configuration  menu  allows  you  to  set auto-dial destinations. These

destinations can be telephone numbers if  your  PC  is  connected  to a

modem, or  COMMIX port  names or  numbers if  your PC is connected to a

COMMIX 32. You can set up  to  four  auto-dial  destinations.  When you

choose Terminal Emulation in the main menu, these four destinations are

shown. You  can  then  simply  choose  one  and  a  connection  will be

automatically dialed  through your  modem or  made through COMMIX. This

will be explained in greater detail in Section 4 of this manual.

The auto-dial destinations are  set  by  selecting  one  of  the preset

destinations  in  the  Terminal  Emulation  sub-menu,  pressing RETURN,

typing a telephone number, or a COMMIX port  number or  name, and again

pressing  RETURN.  To  remove  a  destination from the list, select the

destination, press RETURN,  press  the  space  bar,  then  press RETURN

again.

SAVE CONFIGURATION

Once  you  have  configured  your  CNS  system  it is active during the

current session. If you  want the  information to  be active  after you

reset your PC or toggle power, you must Save Configuration.

ITRON RECOMMENDS  THAT YOU  SAVE CONFIGURATION WHENEVER YOU CHANGE YOUR

CNS CONFIGURATION.

To Save Configuration, simply select the Save Configuration option from

the configuration sub-menu, and press RETURN.

The configuration is kept in a file called CNS.CFG which is in the same

directory as the CNS  program itself.  Do not  place them  in different

subdirectories. When  you Save  Configuration you  DO NOT have to be in

the directory  holding CNS.COM  and CNS.CFG.  When you  first load CNS,

normally via  commands in  AUTOEXEC.BAT during system boot, the default

directory must be set to the directory holding these files. See section

2 of this manual for more details.

3-5

SECTION 4

USING CNS

INTRODUCTION

Now that  you've configured  COMMIX Network  Software, you are ready to

put CNS to work. When ever you wish to use CNS, simply press ALT-period

key. The  CNS main  menu will  pop up on your screen as shown in figure

4-1. It will not interfere with  whatever else  you are  doing. You can

pop it  up while  using a  spreadsheet, a  word processor, or any other

application. You can, of course, pop it up while at the DOS prompt.

Throughout this section, we'll  talk about  "selecting" and "choosing."

"Selecting" means  moving the highlight bar on a menu to an option. You

can use the up or down arrows to move the  highlight bar.  You can also

type the  first letter of the option and the highlight bar will jump to

it. If more than one option start with the same  first letter,  you may

have to press the letter more than once.

"Choosing" means  pressing the  RETURN or  ENTER key when the highlight

bar is on the desired option.  Depending on  the option,  choosing will

either take  you to a sub-menu, toggle a setting, or cause an action to

occur.

FILE TRANSFER

File Transfer allows you to send and receive files from another PC. The

other PC  may be connected to a modem which CNS will dial, or it may be

part of a COMMIX network to  which your  PC is  attached. You  can also

transfer files  between two  PC's hard wired to each other. All control

is from the PC on which  you are  working. We'll  refer to  this as the

"local PC."  The other  PC will  be referred to as the "remote PC," and

must have COMMIX Network Software installed.  File access  control must

be set on the remote PC as discussed in section 3 of this manual.

The start  file transfer, select and choose File Transfer from the main

COMMIX Network Software menu and you'll see the  file transfer  menu as

shown in  figure 4-2.  Before transferring a file you need to set three

things:

REMOTE MACHINE -- If your  PC is  cabled to  a modem,  this is the

telephone number of the remote machine. If it is

cabled to a COMMIX  32 this  is the  COMMIX port

name or number of the remote PC. If two PC's are

directly cabled to each other, no REMOTE MACHINE

should be specified.

READ FILE --      The name  of the file to transmit. The read file

is on the local machine if you are  sending from

local to  remote. It is on the remote machine if

4-1

you are receiving from the remote and sending to

local.

WRITE FILE --     The  name  the  file  is  to  have on the target

machine.

You can use the asterisk as a wildcard character just as you would with

DOS. The  question mark  can also  be used  as a  wildcard for a single

character as it is in DOS.

For example you wish  to send  all files  in the  \FINANCE directory of

your  PC  with  a  file  extension  of  ".WKS"  to  another PC with the

telephone  number  722-1234.  On  the  remote  machine  you'd  like the

filenames  to   have  the  extension  ".BAK"  and  be  in  the  \BACKUP

subdirectory.

Set the REMOTE MACHINE to "7221234" . Set READ FILE to "\FINANCE\*.WKS"

and the WRITE FILE to "\BACKUP\*.BAK" .

Now you're  ready to  transmit, that  is, to send from the local to the

remote machine. To begin the transmission, select and  choose the field

labelled:

Send file(s) to remote:

A status  screen will  automatically "pop up". This screen will let you

see COMMIX trying to  establish the  connection to  the remote machine.

The status  screen provides  a field at the bottom that allows users to

cancel the dial attempt.

Once the connection is established, a new pop-up will appear to let you

know how  the file  transfer itself is taking place. This file transfer

status screen will also allow  users  to  abort  the  file  transfer if

necessary.

When the  transfer is  complete, a  final pop-up  will appear to verify

that the transfer was successful.

You can receive files from the remote machine in much the same  way. To

begin the transfer from the remote machine, select and choose the field

labelled:

Receive file(s) from remote:

Again, a set of pop-up screens will appear to keep you informed  of the

status of your transfer.

FILE SERVICE

When setting up the configuration menu of CNS as described in section 3

of  this  manual,  you  were  able  to  establish  a  series  of access

4-2

privileges (read  only, write  only, read  and write, no access) to the

files on your PC.

While  the  configuration  menu   allowed   you   to   establish  these

parameters,  CNS  does  not  automatically implement them. These access

parameters are implemented only when the File Service  field is  set to

ENABLED. Select  this field  and press RETURN to toggle between ENABLED

and DISABLED.

If File Service is DISABLED,  there  will  be  no  access  available to

remote users for files on the local PC.

PRINTER

The Printer  option of  CNS allows  users to  divert a printout request

from a  parallel printer  port (LPT1,  LPT2 or  LPT3) to  a serial port

which  could   be  connected  directly  to  a  printer  with  a  serial

interface, to a modem which will dial a  connection to  a printer  at a

remote location, or connected to a COMMIX 32 network.

The printer field of the CNS menu allows users to do the following:

1.   Define the parallel printer port (LPT1, LPT2 or LPT3) to which the

PC typically directs printer output.

2.   Choose from one of  a  maximum  of  six  printers  defined  in the

Printer Setup  field. As  explained in  the configuration section,

the six choices can be  different  printers,  or  several printers

with different logical names to define initialization strings that

set different print modes or any combination of the above.

CNS will divert the  printed output  from the  designated parallel port

and to divert it to the serial port named in the COM PORT SELECTION AND

SPEED field of the Configuration menu.

To define the parallel port from which CNS will divert data, select the

PRINTOUT FROM:  field of the Printer menu screen. The RETURN key allows

you to select LPT1, LPT2 or LPT3. If you want  to keep  CNS active, but

disable the printout diversion, choose DISABLE.

If the  feature is enabled, select and choose the logical printer name,

from the list of six, to which you want CNS  to redirect  the printout.

Once users  choose the  printer, the DIVERTED TO: field should indicate

the change. To double check that CNS recognized the change,  go back to

the Main  CNS screen.  The PRINTOUT  FROM: and  the DIVERTED TO: fields

should show the same selections as the printer menu screen.

The COMMIX Network Software printer setup required is simpler than that

for DOS (no need to use the DOS MODE command). Only three wires need be

used: transmit and receive data and ground. None  of the  control lines

need be  connected on  the IBM,  and the  printer should  be set up for

4-3

XON/XOFF (DC1/DC3) software flow control.

TERMINAL EMULATION

The terminal emulation menu causes  your  PC  to  emulate  a  DEC VT100

terminal via one of three services:

1.   Select  one  of  the  four destination addresses that were entered

into the  program  in  the  Preset  Dial  Numbers  section  of the

Configuration menu.

2.   Define and connect to a temporary destination address.

3.   Have  your  PC  enter  terminal  mode  with  no preset destination

address.

To access the Terminal Emulation menu,  select and  choose the TERMINAL

option of the main CNS menu. The Terminal menu screen will appear. This

screen will display the destination addresses (up to  4) stored  in the

configuration menu. This screen will also display the fields that allow

you to define and  connect to  a temporary  destination and  to go into

terminal emulation mode without defining a destination.

To connect  to one  of the  stored destinations, just select and choose

the destination address to which you  would like  to connect.  CNS will

display another  pop-up screen which will display status information as

CNS attempts to make the connection. The display  allows you  to cancel

the connection request simply by entering RETURN.

The CNS program allows users to define a temporary destination address.

This feature is when you  want  to  request  connection  to  a specific

destination several  times during  the course  of a day but will not be

connecting to the destination often enough  to warrant  it being placed

in the CNS configuration file.

To  enter  the  temporary  destination,  select  and  choose  the  TEMP

DESTINATION field of  the  Terminal  Emulation  pop-up.  CNS  will then

display a  window that allows users to enter the temporary destination.

The destination may be a maximum of 18 alphanumeric characters and is a

telephone number or COMMIX port name.

Once the  temporary address  is placed in CNS, you can connect to it by

selecting and choosing the Connect to  Temp field.  Once this  field is

chosen, CNS  will deal  with the connection request in exactly the same

way as it does for permanently stored destinations.

You can also turn your PC  into  a  VT100  compatible  terminal without

connecting to  a pre-defined  destination by selecting and choosing the

Direct Terminal (No Dial) field.

Once your PC is  in terminal  emulation mode,  the monitor  screen will

4-4

clear and  a bar will appear across the bottom of the screen giving you

three special options:

1.   DISCONNECT AND  EXIT. Pressing  Function Key  9 causes  a modem or

COMMIX 32  connection to  be broken, CNS leaves terminal emulation

and returns to the main CNS menu.

2.   Break the  connection  but  stay  in  terminal  emulation  mode by

pressing Function Key 10.

3.   Hold. Pressing  Function Key  5 will  toggle between screen scroll

and no screen scroll.

You PC's keys will act like those of a VT100 in the following manner:

VT100 KEYPAD KEYS             PC KEYS

1 through 9                   Keypad 1 through 9

Function keys 1 through 4     Function keys 1 through 4.

Arrows                        Arrows

Minus                         Minus

Comma                         Keypad asterisk

Period                        Period

Enter                         Keypad Plus

4-5

SECTION 5

TROUBLESHOOTING

GENERAL CONCERNS

This section will describe several potential difficulties you  may have

using COMMIX Network Software and will tell you how to solve them. Most

often these problems are caused by improper configuration of CNS.

Here are some general settings you should check before going  into more

detailed troubleshooting:

1)   Make sure  that CNS is set to the proper speed for the devices you

are using, such as printer or modem (COM Port Selection  and Speed

sub-menu of the configuration menu).

2)   Verify that  CNS is  using the  proper COM port of you PC, such as

COM1: or COM2: (COM  Port  Selection  and  Speed  sub-menu  of the

configuration menu).

3)   If you  are communicating  with another  PC verify that it is also

using a properly configured CNS.

4)   If you are cabled directly to another PC,  use a  cross-over cable

as discussed in section 2 of this PC.

5)   Verify that  your modem is a Hayes-compatible modem using the "AT"

commands and is properly configured to operate with  your PC. (See

modem documentation.)

MENU DOESN'T POPUP WHEN YOU PRESS ALT-PERIOD

Check your AUTOEXEC.BAT file. The first set of commands should load CNS

as described in section  2 of  this manual.  CNS should  be loaded into

your system before any other memory resident utilities.

If CNS  'beeps' when you press alt-period then CNS is correctly loaded,

but is  unable to  popup. There  are two  common reasons  for this: the

operating system  is busy  (e.g. a  disk format), or graphics are being

displayed.

WON'T DIAL CALLS

Verify the dialing procedure  is correct  for your  application (touch-

tone,  pulse,  COMMIX  port  name).  See  section  3 of this manual for

details.

Verify that your modem is configured properly for  your PC.  Check your

modem documentation for more details.

5-1

GARBAGE PRINTED ON PRINTER

If operating via direct wire or modem, verify that CNS and your printer

are operating at the same speed, and that the printer is set  for eight

bits, no  parity, 1 stop bit. If you are going through COMMIX 32, speed

and parity conversion will be performed.

Consult your printer manual to see if any special initialization string

must be  sent to  the printer.  If this is so, place this string in the

INIT STRING printer parameter of CNS as described in section  3 of this

manual.

WON'T TRANSFER FILES

CNS must  be operating  in both  PC's. If  you are  operating through a

modem, the REMOTE MACHINE NAME must  be  the  telephone  number  of the

modem cabled to the remote machine.

NOTHING HAPPENS OR MUTILATED DATA IN TERMINAL EMULATION

If operating  via direct  wire or  modem, verify  that CNS and the host

machine are operating at the same speed, and  that the  host machine is

set for  eight bits,  no parity,  1 stop  bit. If you are going through

COMMIX 32, speed and parity conversion will be performed.

LARGE LETTERS FAIL TO APPEAR ON PRINTER BANNER

If the printer banner appears as six lines of your banner text in small

letters then CNS has failed to find the character generator tables used

by the machines. This should only happen on poor "clones" of the IBM PC.

5-2