ES10079 Error in PCjr Technical Reference Manual Color Conn.

00/00/00

PCjr Color Display				   (4863001)

IBM PCjr System Unit/keyboard			   (4860067)

Technical Reference				   (1502293)

The connector  shown on page 3-94 of the Technical Reference

Manual is  mislabeled.  The  colors  are  shown in the wrong

order.  The  correct  color  order  is BLUE, RED, INTENSITY,

GREEN.  These colors correspond with pins 5,3,6, and 4.

*** END OF DOCUMENT  ***

AH10226 How to find out how much memory is on the system

11/11/85

BASIC Interpreter by Microsoft			   (6025010)

KEYWORDS: INFORMATION

The amount  of  memory  on  the  system unit may be found by

using the following line:

10 DEF SEG=0

20 MEM=(256*PEEK(&H414))+PEEK(&H413)

*** END OF DOCUMENT  ***

WY10032 Compact Printer Graphics Screen Dumps

12/12/85

PCjr System Unit/Keyboard		  (4860004, 4860067)

Compact Printer					   (5181001)

PCjr Color Display				   (4863001)

KEYWORDS: INFORMATION PROBLEM SOLUTION

To allow  the  contents  of  the  PCjr  color  display to be

printed on an IBM Personal Computer Compact Printer you must

use Dos  3.0  or above.  Load Graphics.Com with the

Compact parameter,  then  you  can  do graphics screen dumps

using the compact printer.

*** END OF DOCUMENT  ***

Note :  This modification or recommendation has not been subjected to

exhaustive testing and may not, in all circumstances, provide

a complete solution.

AH10233 Control-break - example of changing interrupt vector

11/11/85

BASIC Interpreter by Microsoft			   (6025010)

PCjr BASIC (Cartridge)				   (6024101)

KEYWORDS: INFORMATION

Following   is  a  program  that  demonstrates  changing  an

interrupt  vector  from  BASIC.  The vector being changed is

the Control-break interrupt vector (interrupt number 1B, the

Keyboard  break  interrupt) located at hex address 6C-6F, or

decimal   108-111.   The  vector  will  be  changed  to  the

Dummy_Return   vector,  which	effectively   disables   the

Control-break	key.  Please  see  the  Technical  Reference

manual  for  your  system  unit  for  more information about

interrupt vectors.

10 CLS:DEFINT A-I

20 DIM A(4)

30 DEF SEG=0

40 FOR I=1 TO 4

50 A(I)=PEEK(107+I) 'Save old interrupt vector

60 NEXT I

70 LOCATE 10,18

80 PRINT "Press any key to inhibit control-break";

90 A$=INKEY$:IF A$="" THEN GOTO 90

100 POKE 108,83 'Set to dummy_return vector..

110 POKE 109,255

120 POKE 110,0

130 POKE 111,18

140 LOCATE 15,18

150 PRINT "Control-break now inhibited - try it";

160 LOCATE 16,27

170 PRINT "(Press spacebar to exit)";

180 A$=INKEY$:IF A$=" " THEN 190 ELSE 180

190 FOR I=1 TO 4

200 POKE 107+I,A(I) 'Reset old vector

210 NEXT I

220 LOCATE 20,20

230 PRINT "Control-break is back on - try it";

240 GOTO 240

*** END OF DOCUMENT  ***

TGW0049 PCjr with internal modem and serial printer

07/17/85

PCjr Internal Modem (8600008)

Compact Printer (5181001)

KEYWORDS: PROBLEM SOLUTION INSTALLATION INFORMATION

If an Assistant Series  program  is  used on a PCjr that has

the IBM Internal Modem and the IBM Compact Printer installed

then option 1  (Select  Printer)  in SETUP is used to select

printer type "9  Other"  and  COM2:  to  indicate the serial

printer port is  the  selected  output device.  Use option 2

(Set up serial  card)  in  SETUP  to  initialize  the serial

printer port with these parameters:

Baud Rate = 1200

Parity	= none

Data Bits = 8

Stop Bits = 2

XON/XOFF  = n

*** END OF DOCUMENT  ***

EMS0017 REDIRECTION OF LPT1.  PCJR INTERNAL MODEM REDIRECTED

10/23/85

IBM PCjr  (4860067)

A base model PCjr with no internal  modem  and  no  parallel

printer  adapter  automatically  sends  output  intended for

LPT1:  to the serial port (for the Compact printer).  Hence,

the  use  of  the  serial device is transparent to the user.

This serial port is also COM1:  with no  options  installed.

If  an  internal  modem  is  installed  the serial port then

becomes COM2:.  Under these conditions the printer output is

then  sent  to the COM2:  serial port (the Compact printer).

This is handled automatically by ROM BIOS.

If a  parallel  printer  adapter  is  attached,  the  output

intended  for the printer is automatically sent to the newly

attached parallel printer.   To  use  the  Compact  printer,

output must be redirected to the serial port.

MODE COM1:1200,N,8,2,P

MODE LPT1:=COM1:

The  above  two  DOS  commands  work  on  a  PCjr without an

internal modem.  COM2:  is used when the modem is installed.

*** END OF DOCUMENT  ***

To All Jr Owners:

Here is a packet of information I received from IBM's

main technical support database system. There are a few

other notes regarding obscure programs on the Jr. like

Homeword, I didn't bother to include them.

There are a few files that are not specific to the jr.

but they apply anyway.

Good luck with your jr's

Sincerely,

Charles Eicher

Computerland of Studio City

11313 Ventura Blvd

Studio City, CA 91602

EMS0030 PCjr Keyboard scan codes different

11/01/83

IBM PCjr System Unit/keyboard			   (4860067)

Single character scan codes are identical.

The extended scan codes  released  by  the PCjr keyboard are

slightly different than the PC's. If the Fn key is not

released before pressing the f7-10 keys the following is tru

**Shift  F9  gives no scan code on the PCjr.

**Ctrl F8, Ctrl F9, and Ctrl F10 give no scan code.

**Alt F7 also results in no scan code being released.

The shift print screen works just like the PC.  The  Control

Print Screen is replaced by Fn/Echo.

*** END OF DOCUMENT  ***

JC20024 PCjr Time of Day Clock Gains Time.

07/16/85

IBM PCjr System Unit/keyboard			   (4860067)

Keywords: Problem Information

The PCjr time of day  clock  gains  time  each  time  a disk

access is performed.  This  is  a  known BIOS problem and at

this time a solution has not been reached.

*** END OF DOCUMENT  ***

GB10005 PCjr Serial Port

07/17/85

UNIT-SYSTEM JR		       (4860067)

KEYWORDS: PROBLEM REUUIREMENTS SOLUTION

The maximum  speed  for  the  ASYNC port on the PCjr is 4800

baud.

This  may  make  it difficult to install some serial devices

unless  the  devices  have  the  capability to adapt to this

speed.

*** END OF DOCUMENT  ***

GB10007 PCjr Function Key Usage

07/17/85

IBM PCjr System Unit/keyboard			   (4860067)

KEYWORDS: INFORMATION PROBLEM SOLUTION

PCjr function keys may appear not to function if the correct

procedure is  not  followed.  The proper use is to press and

RELEASE the  Fn  key  before pressing a numeric key.  In the

case of extended codes the shift or the alt key is held down

while pressing  and  RELEASING  the  Fn key then while still

holding the shift or alt key depressed, pressing the numeric

key. The keyboard definition chart in the PCjr GTO states to

press and  hold  certain  keys  but  does not say to release

others.

*** END OF DOCUMENT  ***

JC20007 Use of ALT key to enter extended codes on the PCjr.

07/17/85

UNIT-SYSTEM JR		       (4860067)

KEYWORDS: INFORMATION PROBLEM

This  document  is  to clarify the use of the ALT key on the

PCjr  keyboard when entering the extended codes.  When using

the  ALT  key to enter the extended codes, the keyboard must

be  in Numeric Lock first.  To put the keyboard into Numeric

Lock,  hold  ALT and press FN, then N. (This is a toggle, to

take  the  keyboard  out  of  Numeric  Lock, repeat the same

procedure.)  Once  the  keyboard is in Numeric Lock, all 256

extended  codes  may be entered by  holding down the ALT key

and  then  typing in the numeric value of the extended code.

See  page 4-9 in the Guide to Operations Personal Computer -

PCjr manual, PN:1592292.

*** END OF DOCUMENT  ***

MV10124 Using a PCjr as a terminal on XENIX

07/20/85

PC XENIX OPERATING SYSTEM      (6024207)

UNIT-SYSTEM JR		       (4860067)

KEYWORDS: INFORMATION SOLUTION INSTALLATION REQUIREMENTS

A PCjr can easily be attached to a system running XENIX as a

"dumb"  terminal.  A  PCjr  is  a  good  choice  because  in

addition to its use  as a terminal, it can also be used as a

stand alone computer.

For  relatively low cost, a user can have both a terminal

and a powerful computer.

*** END OF DOCUMENT  ***

DLE0026 Non-Maskable Interrupt problem in DOS 2.1

08/02/85

IBM PCjr System Unit/keyboard			   (4860067)

DOS Disk Operating System Ver. 2.0 (6024061) & 2.1 (6024120)

KEYWORDS:  PROBLEM INFORMATION UPDATE

There is a problem in DOS 2.1 that may  manifest  itself  by

hanging  the system after a keystroke in any application and

will be restricted mainly to the PCjr.

A  patch  to  correct  the  problem  is available on the DOS

1.1/2.0/2.1  PATCH  Diskette.  This diskette is available on

request from the Dealer Support Center.

*** END OF DOCUMENT  ***

RS10026 Program Languge Compilers on the PCjr

08/07/85

FORTRAN Compiler Version 1.0			   (6024012)

Pascal Compiler Version 1.0			   (6024010)

IBM PCjr System Unit/keyboard			   (4860067)

FORTRAN Compiler Version 2.0			   (6024127)

Pascal Compiler Version 2.0			   (6024128)

KEYWORDS: INFORMATION INSTALLATION REQUIREMENTS

Yes,  the language  compilers  can run on the IBM PCjr.  You

must  take  several  factors  into consideration.  Lets list

these first.

1.  Both  of the version 2.0  compilers  require  160KB  of

memory for DOS 2.0 and 190KB for later versions of DOS.

2. Both of the version 2.0  compilers  require  two diskette

drives.

The IBM Software Guide(dated September 1984) stipulates that

the  above  compilers "Needs extended memory with the PCjr".

If  you  look  at the MINIMUM SYSTEM REQUIREMENTS section of

the  same  brochure,  the FORTRAN, and PASCAL entries states

that  the  IBM PCjr requires 512KB to apportion between user

memory and RAM disk".

The  512K  breaks  down  as follows:  approximately 32KB for

DOS,  16KB  for  Video  Buffer,  160KB  for  the  particular

compiler,  and  314KB  for  a  RAM disk.  In this manner the

requirements   are   met,  and  the  compiler  will  execute

properly.

*** END OF DOCUMENT  ***

EF10144 Screen intensity

09/05/85

IBM PCjr BASIC (with Cartridge)  Version 1.0	   (6024101)

IBM PCjr System Unit/keyboard			   (4860067)

KEYWORDS: INFORMATION

ThePCjr  provides  for full contrast/brightness on all types

of monitors  when  it  is running with DOS 2.1 but, when the

machine goes  into  BASIC  either directly or via a program,

the  screen  intensity  drops to approximately half.  On the

display,  full gain on contrast and brightness controls does

not allow  you  to regain a normal brightness.  This is true

in Cassette or cartridge BASIC.  The intensity is  purposely

dimmed so  that  it  does  not overdrive an RGB or composite

monitor.  To  get  full  brightness on the display enter the

following BASIC statement:

Screen 1,0

This statement will set your screen  to high intensity.

*** END OF DOCUMENT  ***

JC20006 PCjr Cartridge Basic Screen Mode 5 or 6

10/11/85

IBM PCjr BASIC (with Cartridge)  Version 1.0	   (6024101)

IBM PCjr System Unit/keyboard			   (4860067)

PCjr Power Expansion Attachment			   (8600006)

KEYWORDS: PROBLEM SOLUTION INFORMATION

Basic  Statement,  SCREEN,  Modes  5  or  6 are supported in

Cartridge  Basic  only  and  require  a  minimum  of 128K of

memory.   If   you  have  not  issued  the  Basic  Statement

CLEAR,,,32768  prior  to  issuing  SCREEN,  Mode  5 or 6, an

Illegal  Function  Call  will  result.  See  Basic  Personal

Computer  -  PCjr  Manual (6024101), for the CLEAR statement

and for the  SCREEN statement.

*** END OF DOCUMENT  ***