All current Zenith 286 and 386 desktop systems (except

for the Z-286/25) are equipped with a system whereby slow ROMs

are copied to high speed RAM to increase performance. The system

is refered to as the slushware system, since it is a hybrid of

software (RAM) and firmware (ROM). Other manufacturers use a

similar scheme and refer to it as "shadow RAM."

Our systems use slushware for two different things: BIOS

(monitor) ROM slush and video ROM slush. It is impossible to

disable BIOS slush because the actual ROM is located just shy of

the 16MB line. DOS and other real mode applications are limited

to 1MB in their addressing capabilities. The ROM is copied to

some write-protected RAM at the top of the first meg, where the

ROM is traditionally located. There is no way an application can

know that there is not a ROM at that address--it can guess by

seeing t can be read faster, but that cannot affect operations.

The only potential problem with BIOS slush comes when you have

16MB of RAM in a system, or use a device that locates itself in

that neighborhood--like the ALL Chargecard.

Video slush is slightly different. Video cards with ROMs

(EGA and VGA) use the C0000H paragraph for their code. EGA ROMs

usually extend from C0000-C3FFFH; this is true for the Z-449. VGA

ROMs are slightly longer; they extend fro m C0000-C7FFFH. This

applies to most VGA cards including the Z-549. The Z-449 and

Z-549 cards have a ROM which is designed to be "slushware aware."

These ROMs have a special signature that tells our BIOS (monitor)

ROM that the code should be copied into slushware. EGA and VGA

cards from other vendors lack this signature and will not be

slushed. MDA and CGA cards don't have their own ROM code (they

use the system BIOS) and therefore are not slushed.

The area of memory from E0000-E7FFFH is reserved for

video slush. Regardless of the type of video card used, this

block of memory is used by write-protected RAM which cannot be

disabled. As a direct result of this, even when video slushware

is not used, those addresses are not available to any hardware

card.

The device driver UNSLUSHV.SYS does not disable video

slushware; all it does is make sure that it is not used. this

makes it possible for programs which use 386 memory mapping

techniques (like LIMSIM, QEMM, 386-to-the-Max) to use E0000H as

an EMS page frame or "high memory" for loading TSRs. These

programs don't care what is physically there, they just want to

use the addresses. The reason these programs fail when slushware

is used is that applications are expecting video BIOS to be at

E0000H, and they crash when they try to execute the EMS page

frame or other data that's been put there.

Disabling video slushware makes DOS video performance

suffer. Scrolling is slowed considerably. Still, most

graphics-intensive DOS applications (like Windows) bypass BIOS

and control the video hardware directly, so they run just as fast

as they normally do.

To summarize: BIOS slushware cannot be disabled. Doing so

would prevent the CPU from accessing the ROM code it requires to

operate. Video slushware can be disabled to the point where it is

no longer used, but will still be a block of write-protected RAM

from E0000-E7FFFH. This block will conflict with any other

hardware devices using those addresses. But if the block is

unused, software packages which use 386 memory mapping tricks can

use E0000-EFFFFH for EMS page frames and whatnot.

Using the driver: The command to use the driver is

"device=unslushv.sys." Just make that the first line in your

CONFIG.SYS file and put the driver in the root directory. That

will prevent the video card from using video slush.

Technical info: the driver changes the interrupt vectors

for INT 10H and 6DH so that they use C000H as the segment instead

of E000H.