H E R C S A V E   1 . 3

Hercules Graphics Card (tm) Screen Blanker

Version 1.3

(C) Copyright 1988,89 - Benjamin Diss

Limited permission is granted to individual users for product

evaluation and testing.  Continued use of this shareware program

requires registration as described later in this document.

All other rights are explicitly reserved by the author.

- INTRODUCTION -

Images that appear on a computer monitor for an extended period of

time may burn the phosphors of the monitor.  This leaves a ghost image

on the screen.  To help eliminate this problem a hoard of screen

blanking utilities have been distributed.  These programs install

themselves in memory and monitor keyboard activity and sometimes even

video activity.  When there is no activity they blank the screen,

assuming that the user has no immediate need for it.

Unfortunately, none of these programs work properly with the Hercules

Graphics Card (HGC) while in graphics mode.  When these programs

restore the screen they assume the video card is IBM compatible and

treat it as such.  This results in a screen full of "garbage"

characters.

HERCSAVE eliminates this problem by working within the limitations of

the HGC.  It allows screen blanking in both text and graphics modes.

(1)

- USAGE AND INSTALLATION -

HERCSAVE operates by turning the display off after a user specified

period of keyboard inactivity and on when a key is pressed.  HERCSAVE

does not monitor video activity as some other blankers do.

HERCSAVE can be installed by typing the following at the command

prompt:

HERCSAVE s [/o]

s -  represents the number of seconds of keyboard inactivity before

blanking.  It is a mandatory entry.

/o - causes the program to use the optional retrace blanking routine.

The time before blank may be changed after the program is installed by

entering the command again with the new seconds.  The blanking method

cannot be changed once HERCSAVE is installed.

HERCSAVE supports HOT-KEY blanking.  Once installed, pressing

<ALT>-<LEFT SHIFT> will instantly blank the screen.

There should be no need to use the optional blanking method.  Install

the program in your system and use it.  If you notice anything unusual

after the screen restores in graphics mode you may want to try the

optional method.

The program is intended to be run from the autoexec.bat file.

HERCSAVE is a well behaved resident program so it can be loaded in any

order with other resident programs.  Use your favorite text editor to

place the appropriate entry into your autoexec.bat file.

(2)

- TECHNICAL CONSIDERATIONS -

The screen can be blanked and restored using two methods.  The default

is to set the vertical size to zero to blank and restore the size to

restore the screen.  The optional method turns the vertical retrace

off to blank and on to restore the screen.

The program installs only the required code.  Any routine not needed

is not in resident memory after installation.

The program copies the required blanking and restoring procedures to

the resident code space.  It redirects only two interrupts and uses

minimal memory by releasing the environment space before going

resident.  Version 1.3 requires only 496 bytes of resident memory.

History:

4-2-89    Version 1.3 introduces SafeWare!  HERCSAVE now checks its

file image against a pre-caclulated 32 bit CRC.  This warns

of viril infection.

Elaborates on the light pen register tests and no longer

aborts after failing.

3-4-89    Version 1.2 fixed a bug in the hot key routine (thanks to

John Lin) that caused some toggle keys (Ctrl, Shift, etc.)

to appear "stuck" after restoring.

Added test to check for proper light pen register responses.

Program aborts if this test fails.

12-8-88   1.1a changed the default blanking method to 'verticle size'.

(3)

- REGISTRATION AND DISTRIBUTION -

The general public is encouraged to share and distribute copies of

this program free of charge provided the distribution file is kept

intact and all files are unaltered.  Users who find HERCSAVE useful

and intend to use it must register their copy.  Registration entitles

the user to a current version of the program, technical support, and

product upgrades.

Support from users allows the author to develop additional features

and future versions of the program.  Any comments, either good or bad,

are welcome as user feedback stimulates the design process.  Your

payment of $15.00 would be greatly appreciated.  Send payments and

other correspondence to:

Benjamin Diss

The Smart Solutions Group

5800 Oakwood Dr.

Suite 1B

Lisle, IL  60532

The author may also be reached on CompuServe and GEnie:

CIS# 71270,341

GEnie - BDISS

The assembly language source code, site licenses, and custom versions

are available from the author for a modest fee.

Hercules, Hercules Graphics Card, and HGC are registered trademarks of

Hercules Inc.

IBM is a registered trademark of International Business Machines Inc.

(4)