HERCSAVE 2.0

Hercules Graphics Card (tm) Screen Blanker

----------------------------------------------------------------------

(C) Copyright 1988,89 - Benjamin Diss/The Smart Solutions Group

Limited permission is granted to individual users for product

evaluation and testing.  Continued use of this shareware program

requires registration as described later in this document.

All other rights are explicitly reserved by the author.

-- INTRODUCTION ------------------------------------------------------

Images that appear on a computer monitor for an extended period of

time may burn the display phosphors.  This causes permanent damage to

the monitor and severely limits it's life span.  After continual

damage the monitor eventually becomes useless.

To eliminate this problem, programs have been developed that observe

the computer system and blank the screen under certain conditions.

These programs install themselves in memory and monitor keyboard

activity.  When there is no activity for a predetermined amount of

time, they blank the screen, assuming that the user has no immediate

need for it.  These programs work fine for IBM compatible video

systems, however they do not work for Hercules video cards.

When Hercules designed the original monochrome graphics card, there

was no IBM standard to follow.  It was not possible for Hercules to

build an IBM compatible monochrome graphics card.  Therefore, the IBM

BIOS does not support the Hercules Graphics Card (HGC) and normal

screen blankers do not work properly.

HERCSAVE recognizes the limitations of the HGC.  It does not rely on

any IBM BIOS or DOS functions.  It programs the HGC's hardware

directly allowing it to safely blank and restore the screen in both

text and graphics modes.

-- SYNTAX ------------------------------------------------------------

HERCSAVE operates by monitoring the keyboard.  After a specified

period of inactivity the program will blank the screen.  Any keypress

will then restore it.

Syntax: HERCSAVE /Tseconds [options]

/t      TIME before screen blanks in seconds.  This option must be

specified to install the program as there is no default.  To

disable time-out blanking specify zero by entering /T0.

/h      HOT-KEY to force blank where:

1=Right Shift

2=Left Shift

4=CTRL

8=ALT

Select any combination of these toggle (shift) keys by adding

the desired codes together.  The default is <ALT><Left Shift>

or /H10.  To disable hot-key forced blanking specify zero by

entering /H0.

/o      OPTIONAL retrace screen blanking method.  The default method

is vertical size modification.

/u      UNINSTALLS or removes the program from memory.

The TIME and HOT-KEY options may be changed after the program is

loaded.  This allows the temporary disabling of HERCSAVE for

conflicting programs.

For example, "HERCSAVE /T300 /H3" would install HERCSAVE to blank

after about 5 minutes of keyboard inactivity, and would allow a forced

blank by pressing <LSHIFT><RSHIFT>.  HERCSAVE would blank and restore

by modifying the vertical size register.

HERCSAVE is a well behaved resident program so it can be loaded in any

order with other resident programs.  However, if you plan to uninstall

HERCSAVE it must be the last resident loaded that uses the keyboard

and timer interrupts.

HERCSAVE redirects only two interrupts and uses minimal memory by

releasing the environment space before going resident.  Version 2.0

requires only 512 bytes of resident memory for the default blanking

method and 592 bytes for the optional retrace method.

-- COMPATIBILITY -----------------------------------------------------

Since HERCSAVE uses hardware instructions specific to the Hercules

Graphics Card, it tests for true compatibility.  If it determines that

the active video card is not an HGC it tells you so and exits.

If the card responds correctly, HERCSAVE makes two more tests.  These

tests ensure that the card will be able to properly report the video

mode.  The program tests the responsiveness of the light pen

registers.  HERCSAVE uses these registers to determine the number of

pixels currently being displayed.  This number then indicates whether

the video mode is text or graphics.

If either of these two tests fails, HERCSAVE reports and error and

continues with the installation process.  Be aware that if your card

doesn't support these registers, graphics mode programs may not

restore correctly.

-- TECHNICAL ---------------------------------------------------------

The screen can be blanked and restored using two methods.  The default

is to set the vertical size to zero to blank and restore the size to

restore the screen.  The optional method turns the retrace off to

blank and on to restore the screen.

- Vertical size blanking -

The vertical size method works best because it is simpler, more

reliable, and uses less resident memory than the retrace method.

To blank using the vertical size method, HERCSAVE tells the HGC

hardware to display the screen with zero lines.  This makes the screen

appear blank.  To restore, HERCSAVE determines if the video mode is

text or graphics.  It then tells the HGC to display the screen with

the appropriate number of lines based on the current mode.

The only limitation to this method is that on some systems when the

screen blanks, the top row of pixels continues to be displayed.  This

is a very minor flaw in the hardware of your card and is only a

cosmetic limitation.

- Retrace blanking -

The retrace method is more difficult.  To blank the screen HERCSAVE

tells the HGC to turn off the light gun (in the monitor).  This blanks

instantly, without leaving a line remaining at the top of the screen.

Restoring is much more complicated.  To do this HERCSAVE first

determines the video mode.  If the HGC is in graphics mode, it must

next determine the active video page.

It needs the active video page to properly set the Video Mode Control

Port.  This port controls many of the hardware features of the HGC.

Unfortunately, it is limited to "write only" and cannot be "read"

from.  Therefore, the active video page must be determined by some

other method.

Fortunately, Microsoft has established a solution with its mouse

driver.  Graphics based programs that support a Microsoft compatible

mouse must update the Video BIOS Data Area when they change to

graphics mode.  This allows the mouse driver to determine the active

video page so that it may display the mouse pointer.  Since it is not

possible to determine the active video page any other way, HERCSAVE

checks this area and uses the value it finds.

It then calculates the proper Video Mode Control Byte and sends it to

the port to turn the light gun on.

If, when attempting to restore in graphics mode, the screen appears in

random blocks and shapes it can be assumed that your application does

not update the Video BIOS Data Area.  If this is the case with your

system, it probably would be best to use the vertical blanking method.

- Vintage equipment -

Early in 1983, when Hercules first released their monochrome graphics

card, some people tried to use normal MDA (IBM monochrome text) screen

blankers to save their monitors.  Some of these people reported that

their monitors were destroyed when these programs attempted to restore

in graphics mode.  Unless you are using some VERY old equipment this

anomaly should be of no concern to you.

These monitors were damaged because they could not support some higher

display bandwidths and the HGC could not properly report its current

mode.   These programs that caused the damage used a retrace blanking

method similar to HERCSAVE.  The difference is that HERCSAVE uses

different routines to detect the current video mode.

HERCSAVE has been distributed for about 14 months.  I have not

received one complaint that it has caused any damage to any system.

There is no reason to believe that HERCSAVE's retrace blanking

routines may cause damage to your monitor unless it was manufactured

before 1984.  If you are using vintage equipment then do not use the

retrace method.  The vertical size method (the default) is safe to use

for all equipment.

- Virus Detection -

HERCSAVE attempts to detect if a virus or unwanted hacker has altered

the program.  It does this by comparing its file image with a 32-bit

CRC value calculated when the program was compiled.  If a change is

detected HERCSAVE tells you so and exits.

If you receive a copy of the program that will not run because of

this, please contact me at the address below and tell me where you

obtained the program.  I will ensure that the BBS or distributor is

provided with a valid copy.

-- HISTORY -----------------------------------------------------------

7-10-89 Released version 2.0.

7-9-89  Elaborated on the technical notes in the documentation.

6-9-89  Did some additional checking for the current video page in the

retrace method restore procedure.  In well behaved

applications this should help to restore properly.

6-8-89  Squashed a major bug in the logic of the mode detection

routine.  This bug caused some graphics screens to restore

only the top third of the screen and some text screens to

restore with scattered garbage.

Changed from 16 bit port writes to 8 bit.  Some clone cards

did not respond well to the more efficient 16 bit writes.

5-5-89  Enhanced the documentation, the status messages, and the logic

of the install functions.

4-24-89 Changed the time before blank command line option, added the

uninstall command, and changed the organization of routines in

the resident segment.  This reorganization resulted in lower

memory requirements for users of the vertical size blanking

method.

4-20-89 Added the user-specified HOT-KEY.

4-2-89  Version 1.3 introduces SafeWare!  HERCSAVE now checks its file

image against a pre-calculated 32 bit CRC.  This warns of

viral infection.

Elaborates on the light pen register tests and no longer

aborts after failing.

3-4-89  Version 1.2 fixed a bug in the hot key routine (thanks to John

Lin) that caused some toggle keys (Ctrl, Shift, etc.) to

appear "stuck" after restoring.

Added test to check for proper light pen register responses.

12-8-88 1.1a changed the default blanking method to 'vertical size'.

5-10-88 Initial release of Version 1.0

-- REGISTRATION AND DISTRIBUTION -------------------------------------

This program is shareware.  You are encouraged to share and distribute

copies of this program free of charge provided the distribution file

is kept intact and all files are unaltered.  If you find HERCSAVE

useful and intend to use it then please register your copy.

For only $15.00 you can help support the shareware concept.  By doing

so, you allow me to develop and enhance HERCSAVE.  Each successive

version that I have released has been based on the comments of

registered users.  $15.00 is not a lot to ask, considering what it

will save you in the life of your monitor and the time and effort I've

spent developing it.

See the file ORDER.FRM for additional information on registration.

Any comments, either good or bad, are welcome.  Please write, E-Mail,

or call if I can be of any assistance.

Benjamin Diss

The Smart Solutions Group

5800 Oakwood Drive

Suite 1B

Lisle, IL  60532

(312) 963-7975

CIS# 71270,341

Hercules, Hercules Graphics Card, and HGC are registered trademarks of

Hercules Inc.

IBM is a registered trademark of International Business Machines Inc.