===================================================================15 Nov 89==

Speed Video Test  -- Version 1.0  -- Bernardo Zamora

==============================================================================

THE QUEST

---------

A while ago I was wandering around the SIMTEL-20 <screen> subdirectory

and happened to find some screen-speeding programs. I requested all of

them and realized they would indeed speed all access to video (if

BIOS calls were used)....

Sadly I also realized none of them provided means to compare their

effectiveness, and while they claimed speeding most of the BIOS-video

subfunctions, I didn't believe it all...

So I started this quest for truth, to find out the best of all the

video speed up programs.

The testing equipment was an ACER 1000 XT Turbo with CGA, 640 RAM,

30Mb Hard Disk, color monitor, no caches, no other video-intercepting

programs loaded.

Remember that the most important/used/necessary subfunctions are :

* Set cursor position.      (02)

* Scroll window up.         (06)

* Write char and attribute. (09)

* Teletype write.           (14)

The programs should try to improve mostly this four subfunctions.

==============================================================================

THE COMPETITORS

---------------

Name of program    : AD_VIDEO.1

Contained file     : ADVID105.ARC

Version            : 1.05

Date               : 1989

Copyright          : AnDan Software

Way of installing  : Driver in CONFIG.SYS

Special features   : Provides programs to change to 43/50 rows, has 8

different versions with improved performance, can

be told to prevent snow. It's the driver that

improves more functions and performs near the best

in all tests. It proved the fastest (a little

short of viz086).

Drawbacks          : It's the only program that cannot be deactivated.

Memory used        : 1392 bytes

Program size       : 1727 bytes

Shareware          : Shareware, $5 students, $10 private, $40 companies.

Name of program    : THEFAST.COM

Contained file     : THEFAST.ARC

Version            : 1.04

Date               : 1987

Copyright          : TheSoft Programming Services and Ian E. Davis

Way of installing  : TSR

Special features   : Initially a ZENO rewrite, to circunvent bug

with Tandy 1000.

Drawbacks          : Little speed improvement in only 6 subfunctions,

fails to improve scroll-up, it's the program that

speeds up TELETYPE by a smaller amount. Little here

to recommend this one.

Memory used        : 864 bytes

Program Size       : 942 bytes

Shareware          : $5 Shareware (probably not registered)

Name of program    : VIZ086.COM

Contained file     : VIZ153.ARC

Version            : 1.53

Date               : 1989

Copyright          : FM de Monasterio

Way of installing  : TSR

Special features   : Provides switches to 'sleep' the fast program,

and to speed up further with 80286/80386 card. Can

show current video-mode, can be told to change to

any video mode or page (only works in page 0).

Drawbacks          : It looks like a good contender, but has 2 problems :

It doesn't improve a lot the TELETYPE subfunction,

and it FAILED to scroll the screen correctly. When

I typed CLS from the DOS prompt, the screen went

berzerk and thought the screen should only have

1 row, scrolling everything there only. Bad bug!.

The only way to recover was by deactivating it.

Memory used        : 1072 bytes

Program size       : 8544 bytes

Shareware          : Shareware, Noncommercial : $5(8086), $10(80386)

Commercial    : $30 - 10 sites

Name of program    : ZENO13.COM

Contained file     : ZENO13.ARC

Version            : 1.3

Date               : 5/29/1987

Copyright          : ** Several modifications by different authors **

Way of installing  : TSR

Special features   : Provides ASM source file. It has the fastest

teletype speeding (only slowed by the standard

BIOS scroll-window-up function).

Drawbacks          : Fails to speed scroll-window-up. This is one

of the most necessary functions, used in the

TYPE program, in DIR, and in lots of other

common programs.

Memory used        : 976 bytes

Program size       : 805 bytes

Shareware          : (not mentioned anywhere in the manual)

==============================================================================

THE TEST

--------

I decided I needed a NEW test, one that tried all the subfunctions

individually and check their speed increase. This test was called

VTEST, the ULTIMATE test (*smile*). It would confront face to face

the four programs.. and show their strengths and weakneses. This test

performs each subfunction lots of times (see the included PASCAL source

for each test). The timing results are presented here, with an included

final table with all the features highlighted. Type VTEST ? for help.

==============================================================================

THE RESULTS

-----------

TEST           BIOS    ADVID     THEFAST       VIZ      ZENO13

��������������������������������������������������������������������������Ŀ

�0�Set Video Mode        �27.90�27.74 1.01�27.90 1.00�27.52 1.01�27.57 1.01�

�1�Set Cursor Size       �22.41�24.17 0.93�24.22 0.93�25.38 0.88�25.32 0.88�

�2�Set Cursor Position   �33.45�21.59 1.55�19.72 1.70�22.03 1.52�22.30 1.50�

�3�Read Cursor Position  �29.88�19.06 1.57�19.22 1.55�20.71 1.44�20.60 1.45�

�4�Read LightPen Position�19.94�21.70 0.92�22.30 0.89�22.63 0.88�22.63 0.88�

�5�Set active Display Pg �18.95�20.05 0.95�20.93 0.90�21.09 0.90�21.37 0.89�

�6�Scroll Window Up      �66.79�23.56 2.83�66.79 1.00�23.95 2.79�66.73 1.00�

�7�Scroll Window Down    �66.73�23.56 2.83�66.79 1.00�66.73 1.00�66.79 1.00�

�8�Read Chr and Attribute�59.26�21.97 2.70�59.43 1.00�20.71 2.86�59.43 1.00�

�9�Write Chr and Attr    �34.60�14.34 2.41�12.08 2.86�13.35 2.59�13.13 2.64�

�A�Write Character       �44.49�17.85 2.49�15.82 2.81�16.48 2.70�15.60 2.85�

�B�Set Color Palette     �17.96�19.50 0.92�20.05 0.90�20.27 0.89�20.21 0.89�

�C�Write Pixel           �15.05�16.15 0.93�15.71 0.96�16.09 0.94�16.37 0.92�

�D�Read Pixel            �14.45�15.54 0.93�15.10 0.97�15.43 0.94�15.71 0.92�

�E�Teletype write char   �84.31�25.65 3.29�54.21 1.55�46.52 1.81�26.36 3.20�

�F�Get Current Video Mode�17.03�18.62 0.91�10.49 1.62�11.81 1.92�19.39 0.88�

����������������������������������������������������������������������������

Note : all times are in seconds, and points are given as a ratio of

increase/decrease of performing time.

Alas, if an entry says 14.34 2.41

it means the time taken to perform that test was 14.34 seconds, and went

2.41 times faster than the BIOS test.

If the ratio is greater than 1, the test went faster, and if it is

below 1, it went slower. Ratios changes between 0.95 and 1.05 are

irrelevant.

==============================================================================

THE TRUTH

---------

Several programs claimed speeding-up functions they dindn't really

speed at all. Here is the table of all the functions and the way

each contestant performed, also extra information about features

found in these programs is included.

ADVID  THEFAST   VIZ   ZENO13

-------------------------------|-------|-------|-------|-------|---

0  Set Video Mode            |       |       |   .   |       |

1  Set Cursor Size           |       |       |   .   |       |

2  Set Cursor Position      >|   1   |   1   |   1   |   1   |

3  Read Cursor Position      |   1   |   1   |   1   |   1   |

4  Read LightPen Position    |       |       |       |       |

5  Set active Display Pg     |       |       |   .   |       |

6  Scroll Window Up         >|   2   |       |  .x2  |       |

7  Scroll Window Down        |   2   |       |   .   |       |

8  Read Chr and Attribute    |   2   |       |   2   |       |

9  Write Chr and Attr       >|   2   |   2   |   2   |   2   |

A  Write Character           |   2   |   2   |   2   |   2   |

B  Set Color Palette         |       |       |       |       |

C  Write Pixel               |       |       |       |       |

D  Read Pixel                |       |       |   .   |       |

E  Teletype write char      >|   3   |   1   |   1   |   3   |

F  Get Current Video Mode    |       |   1   |   1   |       |

-------------------------------|-------|-------|-------|-------|---

Can be unloaded/uninstalled  |       |   *   |   *   |   *   |

Anything special for EGA/VGA |   *   |       |   *   |       |

Included ASM file            |   *   |       |       |   *   |

-------------------------------|-------|-------|-------|-------|---

(1) = BIOS subfunction acceleration (1.2 - 1.99)

(2) = BIOS subfunction acceleration (2.0 - 2.99)

(3) = BIOS subfunction acceleration (3.0 - 3.99)

(*) = feature provided

(.) = claimed to accelerate but didn't

(x) = failed this test

(>) = very important subfunction

==============================================================================

FINAL WORDS

-----------

I am currently using the AD_VIDEO package, as it presents no

problems and gives impressive performance. I would only like it

to be tweaked for subfunctions 8,9 & A a little bit so it would

beat the other programs in ALL the tests. I recommend giving it

a try, as it really shows you a faster display the cheap way.

==============================================================================

THE TESTER

----------

All ideas, typing and testing done by me :

Bernardo Zamora,

BITNET : PL306971@TECMTYVM (nick "BEAR")

or try contacting friend at

BITNET : EM304599@VMTECMEX

Any comments happily welcome!!

==============================================================================